Txt2Bas- C64/Vic Text To Basic File Converter               Txt2Bas converts Basic programs stored in a listable format on     BBS systems to their true Basic tokenized form. This version was based     on an earlier version by Joel Rubin.               There are faster versions of Text To Basic (/Sequential to Basic)     converters available. The main advantage of this program is that it is     fairly short to type in when the comment statements are left out.               Using a Text to basic converter you can often read Basic programs     written by other computer systems, and saved in a sequential form, and     convert them to true Basic on your computer, even though your computer     uses different tokens other than the computer the program was     originally written on. You can also convert programs that could be     read into your capture buffer, even though you don't have downloading     capabilities.          How does a Basic program get saved in a sequential form? Simply     by listing it to the Disk Drive instead of the printer. (Note that     Txt2Bas may convert such programs properly, although Syntax errors     show up at the end of the program. That is because listing a program     to the disk usually adds an extra marker and a 'Ready' statement.          Here is a simple method to list a Basic Program on the Disk:          Open 1,8,1,"filename,S,W"          Cmd 1:List          Close 1               You can edit out the added carriage returns and 'Ready' statement     in a word processor like Easy Script. (It would have to go through     another conversion to be edited in Speedscript)                 Txt2Bas- C64/Vic Text To Basic File Converter               Txt2Bas works with the C64 and the Vic-20 with either a disk     drive or tape drive. When run the program you want converted should be     on the disk/tape in the drive. Before running the program Poke 186,1     if you are using tape.          The file to be converted should be stored in a sequential form.     Txt2Bas will ask for the filename. Input it. The program will then     prompt " Any Upper Characters (Y/N): ?". Respond yes if the file     appears in Upper case mode when you are in Upper/Lower case mode. Most     PETSCII programs- includeing Hex2Bin- will appear in Lower case mode-     for these you should respond N. Note that Hex2Bin has upper case     characters in quotes that you do not want converted to lower case     form.          Txt2Bas converts the file to Basic form on a line by line basis     using the input buffer. When it is finished it erases itself. If you     have just typed in the program form you shouldn't test it on a     sequential version of itself, unless you have already saved the     program form- it will erase itself, just as it does for any other     program.          Txt2Bas erases itself by changeing the top of Basic pointer in     line 63886. You cannot convert utility programs using line numbers in     the range 63700 up, unless you renumber Txt2Bas, and remove the pokes     in line 63886.          When Txt2Bas finishes the converted program will be listable in     memory, and can then be saved as a normal Tokenized Basic program.               Txt2Bas.Seq is the listable version of this program. Txt2Bas.Bin     is the true Basic form.          Hex2Bin.Seq (Hex2Bin.Bin) is a Basic program that will be useful     to those of you who cannot download programs in Compuserve, but have a     capture buffer. It lets you read the programs into your capture buffer     in a Hexadecimal form (:180000...), and converts them back to their     true form. You can use Hex2Bin to get a copy of CBTerm/4.5 or     CBTerm/4.9- a terminal program with true Xmodem capabilities. Version     4.9 also allows users to Upload & Download even over DataPac.                         Tony Romer