The CompuServe B Plus Protocol                                     May 16, 1988  2:10 PM                                              by                                         Russ Ranshaw

TABLE OF CONTENTS A. Introduction. B. Notation. C. B Plus Packets.    1. B Plus Packet Structure.    2. Quoting of characters.    3. Check Value.    4. B Plus Packet Types.       a) Packet Type `+':  Transport Parameters.       b) Packet Type `T':  File Transfer.       c) Packet Type `N':  Data.       d) Packet Type `F':  Failure.       e) Optional `T' Packets.          1) Packet `Tr' : Download Resume.          2) Packet `Tu' : Upload Resume.          3) Packet `Tf' : Failed CRC.          4) Packet `TI' : File Information. D. B Plus Control Sequences.    1. Enquire.    2. Positive Acknowledge.    3. Wait.    4. Negative Acknowledge.    5. Panic Abort. E. Negotiation of Transport Parameters. F. Terminal Program States. G. Wait for Acknowledge. H. Packet Send Ahead I. Implementation Considerations    1. Time-Out    2. Packet Size.    3. Controlling Excessive Retransmissions. J. B Plus Transport Layer. K. Initiation of a B Plus Session. L. Supporting the Various B Protocol Versions M. Check Value Calculation.    1. Standard B Protocol Checksum.    2. XMODEM-Style CRC-16. N. Interrogation.

A. Introduction.              The CompuServe B protocol was developed in 1981 to provide         support for a special purpose Vidtex terminal manufactured         by the Tandy Corporation.  It was the outgrowth of a         proposed Bi-Sync oriented protocol, but with a different         packet structure and provision for more than even and odd         packets.  The file transfer capability was added in 1982 to         replace the CompuServe A protocol with a more robust         protocol which was in keeping with the over-all B Protocol         design.              Some of the underlying assumptions made in designing the B         Protocol were due to the capabilities of personal computers         which were available at the time.  Such machines were         generally limited in the amount of available memory, 64         kilobytes being a large capacity.  Other factors, such as         the lack of a true UART for data communications, resulted in         the send/wait nature of the protocol where only a single         protocol packet at a time was sent.              The explosive growth of the Personal Computer industry has         given us a plethora of machines, most of which have far         exceeded the early limitations of memory and communication         ability.  This growth has been accompanied by a multitude of         file transfer protocols, such as XMODEM, KERMIT, and ZMODEM.         CompuServe, realizing the need for enhancement, has         developed the B Plus Protocol to meet the increasing demands         being made upon its communication network and host         computers, and to provide added utility for its large family         of users.              As the name implies, B Plus is an extension of the B         Protocol.  In particular:              o  Ability to send multiple packets without waiting for                 individual acknowledgements.              o  Larger data packets (up to 1k at present).              o  Optional use of modified XMODEM CRC-16 check method.              o  Extensions to the standard control character                 quoting.              o  Provision of a mechanism to exchange transport and                 application parameters.                                                  2

B. Notation.              Throughout the remainder of this document the following         notational conventions will be used:                   o  Protocol elements are enclosed within angle                 brackets.  For example:                        <DLE>                 specifies the ASCII code for Data Link Escape.                   o  The C-language notation for hexadecimal values will                 be used.  Thus, the <DLE> character will also be                 shown as 0x10.              Two entities (computers) are involved in a B Plus session.         The term "Initiator" is used to refer to the entity which         initiates the session.  "Responder" refers to the entity         which receives the initiate command.              CompuServe host computers will always be the Initiator in         any communication established with them.              For further information regarding Host support, see Section         "K. Initiation of a B Plus Session."                                                  3

C. B Plus Packets.              1. B Plus Packet Structure.                 B Plus Protocol packets consist of five parts as follows:                      o  Lead-in       <DLE> <B>                 o  Sequence      <0x30 thru 0x39>                 o  Type          <Single byte>                 o  Body          <zero to 1024 data bytes>                 o  Trailer       <ETX> <Check Value>                 The Sequence is incremented by one for each successfully            transmitted packet.  It wraps from 0x39 back to 0x30.  (Note            that 0x30 is the ASCII character 0 and 0x39 is 9.)                 The Sequence, Type, all Body data, and <ETX> are included in            the Check Value.  Note that <Body> and <Check Value> are            Quoted, and that the Check Value is computed on the actual            data bytes rather than their quoted replacements (the            quoting <DLE> is not included).  Hence, if the character            0x13 is to be sent, it is sent as <DLE> <S> but the value            0x13 is included in the Check Value.                 If Standard Checksum is used, <Check Value> consists of a            quoted byte.                 If the XMODEM CRC-16 is used, <Check Value) is transmitted            as two quoted bytes with the most significant byte appearing            first.  By performing the CRC calculation on the XMODEM            CRC-16 value,  the result will be 0x0000 if no errors            occured.                 A sample B Plus packet, sent using the Standard Checksum, is            as follows:                     <DLE>  B    7    T    D    A    S    .    C  <ETX> 0x2A                0x10 0x42 0x37 0x54 0x44 0x41 0x53 0x2E 0x43 0x03  0x2A                 where                     Lead-in   =    <DLE> B                     Sequence  =    7                     Type      =    T                     Body      =    DAS.C                     Trailer   =    <ETX> 0x2A                 If the packet was sent using the XMODEM CRC-16 option, the            Check Value would be 0x57 0xFF instead of 0x2A.                                             4

2. Quoting of characters.                 The B Plus Protocol quotes certain ASCII control characters.            The default set of quoted characters is:                      o  <ETX>  0x03                 o  <ENQ>  0x05                 o  <DLE>  0x10                 o  <DC1>  0x11                 o  <DC3>  0x13                 o  <NAK>  0x15                 The quoting mechanism uses the <DLE> character to prefix the            quoted character.  The quoted character with 0x40 added            follows the <DLE>.  For example, <ETX> is quoted as:                      <DLE>  C                 0x10 0x43                 B Plus also provides a means for the Initiator and Responder            to establish a different set of quoted characters.  This set            may consist of any or all characters in the ranges 0x00 thru            0x1f and 0x80 thru 0x9f.                 Quoted characters in the 0x80 thru 0x9f range are sent as:                     <DLE> ((char and 0x1f) + 0x60)                 Hence, 0x93 is quoted as:                     <DLE>  s                0x10  0x73                 When receiving protocol data, the program should be capable            of decoding any quoted character, regardless of the set of            quoted characters in use.  This must be accomplished as            follows:                     Get a character.                If the character is <DLE>                then                     Get next character.                     If character is less than 0x60                     then                          character = character and 0x1f                     else                          character = (character and 0x1f) + 0x80                                             5

3. Check Value.                 Each B Plus packet contains a Check Value to allow detection            of transmission errors.  The B Plus Protocol supports two            Check Value methods as follows:                 a) Standard B Protocol Checksum.                    The Standard Checksum is calculated as follows:                         o  The checksum is initialized to zero.                    o  For each byte that is to be checksummed:                         -  The old checksum is rotated left 1, with the                            old bit 7 becoming bit 0 in the rotated value.                         -  The new byte is added to the rotated checksum.                         -  If addition of the new byte causes a carry,                            then one is added to the new value.                      b) XMODEM-Style CRC-16.                    B Plus implementations may optionally specify that the               defacto standard XMODEM CRC-16 be used as a check value.               The only departure is that the CRC-16 value is initialized               to 0xffff instead of zero.                      Sample C code for calculating these Check Values is presented in            Section "H. Check Value Calculation."                                                  6

4. B Plus Packet Types.                 The following Packet Types are the minimum required to            support the B Plus Protocol:                     o  `+':  Transport Parameters                o  `T':  File Transfer                o  `N':  Data                o  `F':  Failure                 a) Packet Type `+':  Transport Parameters.                    This is the B Plus Transport Parameters Packet.  It               describes an entity`s Transport capabilities.  The Transport               Parameters Packet is always sent with all characters in the               ranges 0x00 thru 0x1f and 0x80 thru 0x9f quoted, and using               the Standard Checksum.                    The Body of the <+> packet is a series of bytes as follows:                         WS WR BS CM DQ TL Q1 Q2 Q3 Q4 Q5 Q6 Q7 Q8 DR UR FI                    where:                         WS = Window Send.  0x00 implies that a single packet at                         a time can be sent, 0x01 two packets.  The default                         is 0x00.                         WR = Window Receive.  0x00 implies that the entity                         cannot receive other than a single packet at a                         time (that is, requests the other entity to use                         WS = 0x00);  0x01 implies two packets.  The                         default is 0x00.                         BS = Block Size.  This specifies the maximum size of                         the <Body> divided by 128.  Thus 0x04 represents                         512 bytes and 0x08 is 1024 bytes.  The value 0x00                         implies the default size of 512 bytes.                         CM = Check Method.  0x00 is standard checksum; 0x01 is                         XMODEM CRC-16 initialized to 0xffff.  Note that                         CompuServe recommends the use of CRC-16 instead                         of the standard checksum due to its superior error                         detection capability.  The default is Standard                         Checksum, 0x00.                                             7

DQ = Data Quoting Level.  This parameter is part of an                         interim version of the B Plus Protocol.  It has                         been superceeded by the eight byte Quoting Set in                         Parameters Q1 thru Q8 (below).  For historical                         purposes, the Quoting Levels are:                                  0x00:  0x00 0x03 0x05 0x10 0x11 0x13 0x15                             0x01:  0x03 0x05 0x10 0x11 0x13 0x15                             0x02:  0x03 0x05 0x10 0x11 0x13 0x15 0x91 0x93                             0x03:  0x00 thru 0x1f and 0x80 thru 0x9f                              If a Parameters packet contains Q1-Q8, the DQ must                         be ignored and the quoting set established from                         Q1-Q8.  If Q1-Q8 are absent, the above definitions                         must be used.                                            TL = Transport Layer.  The Initiator sets the TL                         parameter to 0x01 when it is running an                                  application that must run under the Transport                         Layer.  If the Responder`s half of the application                         also utilizes the Transport Layer, it must also                         set the TL parameter to 0x01.  The default value                         is 0x00.  More information is in Section "J. B                         Plus Transport Layer."                         Q1-Q4 = A mask indicating which characters in the range                         0x00 thru 0x1f the entity wants quoted.  The                             mapping of bits to characters is:                                Bit:   7    6    5    4    3    2    1    0                           ----- ---- ---- ---- ---- ---- ---- ---- ----                             Q1: 0x00 0x01 0x02 0x03 0x04 0x05 0x06 0x07                             Q2: 0x08 0x09 0x0a 0x0b 0x0c 0x0d 0x0e 0x0f                             Q3: 0x10 0x11 0x12 0x13 0x14 0x15 0x16 0x17                             Q4: 0x18 0x19 0x1a 0x1b 0x1c 0x1d 0x1e 0x1f                                                 CompuServe Hosts currently use:                                  0x14 0x00 0xd4 0x00                         Q5-Q8  Same as Q1-Q4 for the range 0x80 thru 0x9f.                           CompuServe Hosts currently use:                                  0x00 0x00 0x00 0x00                         DR     Download Resume.                                 0x00 : Download Resume not supported                                 0x01 : Supports `Tr' Packet                                 0x02 : Supports `Tr' and `Tf' Packets                                             8

UR     Upload Resume.                                 0x00 : Upload Resume not supported                                 0x01 : Supports `Tu' Packet                                 0x02 : Supports `Tu' and `Tf' Packets                    #####     FI     File Information.  This parameter specifies the                           level of extended file handling supported.                                      0x00 = No extended file handling                                 0x01 = `TI' (File Information) Packet                               These are the currently defined Transport Layer Parameters.               CompuServe reserves the right to define additional               parameters at any time.  Programs should be capable of               accepting any number of parameters.  Received parameters               beyond the known ones must be ignored. If fewer Transport               Parameters are received than the entity understands, then               any beyond the last received parameter must be treated as if               it was provided as 0x00, which must be its default value.                    See #Section "E.  Negotiation of Transport Parameters" for               details on deciding what level of capabilities each entity               is to use.                                             9

b) Packet Type `T':  File Transfer.                    The File Transfer Packet consists of three fields as               follows:                         <Direction> <Data type> <File name>                    1) Function.                       `D' = Download (Initiator -> Responder)                       `U' = Upload (Responder -> Initiator)                       `C' = Close.  End of Upload or Download                       (Note:  Additional "T" packets are defined later.)                    2) Data type.                       `A' = 7-bit ASCII data.  (Allows for End of Line                                                        conversion.)                       `B' = 8-bit Binary data.                       `I' = Image data.  (This is hardware specific data and                             usually consists of some embedded header which                             describes the remainder of the file.  It is                             always 8-bit binary data.)                    3) File Name.                       This field is variable length and specifies the name of the                  file to be transfered.                         A sample File Transfer packet using CRC Check Method is:                       <DLE>  B    7    T    D    A    S    .    C  <ETX> 0x57 0xff                        0x10 0x42 0x37 0x54 0x44 0x41 0x53 0x2e 0x43 0x03 0x57 0xff                    which specifies:                   o    `T':  File Transfer                   o    `D':  Download to the Responder                   o    `A':  This is an ASCII file                   o  `S.C':  The file name                    Note that there are no guidelines as to what the Responder               is to do with the File Transfer packet.  In particular, the               implementor is free to locally verify the request, permit               the file name to be changed, etc.                    If the Responder does not wish to comply with the request               made in the File Transfer packet, a Failure Packet must be               returned to the Initiator.                                             10

c) Packet Type `N':  Data.                    This is a data packet.  The data being transferred is broken               up into a series of <N> packets.  There may be zero to the               negotiated data block size.  A count of zero does not imply               end of file.                 d) Packet Type `F':  Failure.                    This is the general Failure Packet.  A Failure Packet may be               sent at any time; the value of the <Sequence> field is               ignored.  The first character of the <Body> indicates the               general nature of the failure, and the remainder may be a               printable ASCII message further describing the condition.               The following standard failures must be supported:                         `A':  Abort.  Usually sent if the user requests that the                          terminal program cease tranferring data.                         `C':  Capacity failure.  Out of memory or disk.                         `E':  Processing error.  Any error other than described                          by another Failure Packet.                         `I':  I/O error occured.                         `M':  File requested for Uploading is Missing (not                           found).                         `N':  Unimplemented Packet Type was received.                         `S':  Protocol Sequence Number failure.                         `r`:  Transfer Resume failure.                    When a Failure Packet is received, it must be acknowledged and               the transfer aborted.  Similarly, when a Failure Packet is sent,               the sending entity must wait for the acknowledge to arrive.  In               addition, the entity which sends the Failure Packet must ignore               all other packets while waiting for the acknowledgement to               arrive.  Once the Failure Packet is successfully transfered,               both entities are expected to leave protocol mode.                                             11

e) Optional `T' Packets.                    Several "T" Packets may optionally be supported.  The               support for these packets is specified by the DR, UR, and FI               Transport Parameters.                    1) Packet `Tr' : Download Resume.                       When the Responder receives the "TD" (Download Request)                  packet and the specified file already exits, it may attempt                  to resume the download which was aborted during a prior                  session.  In order to do this, both entities must specify                  Transport Parameter DR > 0x00.  If the Responder attempts                  the resumption, a `Tr' Packet is returned to the Initiator                  as follows:                           <DLE> B <Sequence> T r <Length> <CRC-16> <ETX> <Trailer>                       where:                           <Length> = current number of bytes in Responder's                                 existing file.                           <CRC-16> = XMODEM CRC-16, initialized to 0xffff, of all                               existing data in Responder's file.                       Both <Length> and <CRC-16> are transmitted as an ASCII                  decimal string and are both followed by at least a single                  space.                       The Initiator will perform the same CRC-16 calculation over                  <Length> bytes and compare the resulting CRC-16 values.  If                  the calculated and `Tr' values agree, downloading continues                  with `N` Packets and Responder appends the data to the                  existing file.  If the files match exactly, a `TC' (Transfer                  Close) packet is sent.                       If the CRC-16 values do not match, or the Initiator's file                  be shorter than <Length> bytes, the Initiator's response                  depends on the value of the DR Transport Parameter:                               0x01 - A `Fr' (Failed Resume) Packet is sent                          0x02 - A `Tf' (CRC Failed) Packet is sent                       Some operating environments complicate Download Recovery.                  Since the Initiator has no knowledge of any data                  transformations, such as  Carriage Return and Line Feed                  mappings, the Responder must take any such transformations                  into consideration when calculating <length> and <CRC-16>.                  Under some environments, it is probably simpler to maintain                  this information during the download and preserve it in a                  file (Apple Macintosh (TM) for example.)                                                  12

2) Packet `Tu' : Upload Resume.                       The Upload Resume Packet is sent by the Initiator if both                  entities have the Transport Parameter UR > 0x00, the target                  file already exists on the Initiator, and the application                  software has decided to attempt an upload resumption.  It is                  essentially a `TU' Packet with <length> and <CRC-16> fields.                  It is constructed as follows:                           <Lead-in> <Sequence > T u <Data Type> <Length> <CRC-16>                           <File Name> <Trailer>                       where:                      <Data Type> = same as for the `TU' Packet                      <Length>    = number of bytes in existing file on                                    Initiator                      <CRC-16>    = XMODEM CRC-16, initialized to 0xffff, of                                    all <Length> bytes                      <File Name> = same as for the `TU' Packet                       The Responder opens <File Name> and calculates the CRC-16                  for <Length> bytes.  If the resulting CRC-16 agrees with                  <CRC-16> from the `Tu' Packet, it continues by sending `N'                  Packets.                       If the CRC-16 values do not match, the action depends on the                  value of Transport Parameter UR:                               0x01 - A `Fr' (Failed Resume) Packet is sent                          0x02 - A `Tf' (Failed CRC) Packet is sent                       The same operating environment considerations mentioned                  under the `Tr' Packet apply here.                                             13

3) Packet `Tf' : Failed CRC.                       The `Tf' packet is sent when a Resume request fails due to                  CRC failure.  It will be sent only if the Responder and                  Initiator both specify Transport Parameter DR > 1.  It                  implies that the transfer is to continue from the beginning                  of the file.                       Sample packet exchanges for transfer resumption:                               Initiator           Responder      Comments                          ---------           ---------      -----------------                  DR = 0x01 or 0x02:                          TD --------------->                Download Request                             <--------------- Tr             Resume Download                          N  --------------->                CRC matched                                 ...                          TC --------------->                End of transfer                       DR = 0x01:                          TD --------------->                Download Request                             <--------------- Tr             Resume Download                          Fr --------------->                Resume failed                       DR = 0x02:                          TD --------------->                Download Request                             <--------------- Tr             Resume Download                          Tf --------------->                CRC mismatched                             <--------------- ACK            Transfer entire                          N  --------------->                Data                                ...                          TC --------------->                End of transfer                       UR = 0x01 or 0x02:                          Tu --------------->                Upload Resume                             <--------------  N              CRC Matched                                ...                             <--------------  TC             End of transfer                       UR = 0x01:                          Tu --------------->                Upload Resume                             <--------------- Fr             CRC mismatch                       UR = 0x02:                          Tu --------------->                Upload Resume                             <--------------- Tf             CRC mismatch                          ACK--------------->                Transfer entire                                                             FIle                             <--------------- N              Data                                ...                             <--------------- TC             End of transfer                                                  14

4) Packet `TI' : File Information.                       The `TI' Packet is transmitted by the File Sender, provided                  that both entities have their Transport Parameter FI > 0x00.                  It is used to convey specific data concerning the file as it                  exists on the Sending entity.  The format of the `TI' Packet                  is as follows:                           <Lead-in> <Sequence> T I <data type> <compression >                          <file length> <creation date> <creation time>                          <modification date> <modification time>                          <true name length> <true name> <Trailer>                       where:                      <data type> :  A = ASCII, B = Binary.                           <compression> :  Specifies how the file is compressed                                       during transmission (has nothing to do                                       with the content of the file).                                         0x00 = not compressed                                                (other values reserved for                                                 future use by CompuServe)                           <file length> :  Current length of the file being sent.                                       (Actual number of file data bytes being                                       sent, not the number of compressed                                       bytes).  This value is passed for display                                       display or media check only.                                                                 <zone> :           Minutes offset from UCT of Sender.                           <creation date> :  Date the file was created.                                         ASCII decimal string, yyyymmdd                           <creation time> :  Time of day the file was created.                                         ASCII decimal string, seconds                                         since midnight.                           <modification date> :  Date the file was last modified.                                         Same format as <creation date>.                                         (`0' if unknown)                           <modification time> :  Time of day the file was last                                         modified.  Same format as                                         <creation time>.  (`0' if unknown)                                             <true name length> :  Number of bytes in <true name>.                                            (This is a single binary byte.)                           <true name> : The actual file name with any device,                                    directory, or other ancillary information                                    removed.                       It is not necessary that all fields be transmitted.                                        15

A sample TI packet (file information only) is:                          "A?55387 300 19880422 52480 0 0 ?BPLUS.DOC"                       |       |             |       |                      0x00    EST         7:18 PM   0x09                D. B Plus Control Sequences.              The B Plus Protocol defines several Control Sequences in         addition to packets.  Control Sequences are used to manage         the progress of the B Plus Session.              1. Enquire.                 The Enquire Control Sequence consists of the single            character:                         <ENQ>                     0x10                 It is sent in one of two circumstances:                      o  The Responder is in terminal mode and the Initiator                    wants to synchronize its <Sequence> number, and to                    tell the Responder to set up for normal B Protocol.                    When the <ENQ> is received, the Responder must:                           >  Prepare for 512 byte data blocks                      >  Prepare for Standard Checksum                      >  Initialize its Sequence to 0x30                      >  Transmit the string:                                <DLE> + + <DLE> 0x30                           >  Return to terminal mode.  Note:  The terminal                         program  definately should NOT enter B Protocol                         mode when the <ENQ> is received.                      o  The entity sending the packet has received a                    Negative Acknowledge, or timed out while waiting for                    an Acknowledge.  The response to an <ENQ> during a B                    Plus session is a Positive Acknowledge.              2. Positive Acknowledge.                 When a correctly checked and sequenced packet is received,            a Positive Acknowledge is transmitted.  This consists of:                      <DLE> Sequence                 where Sequence is the Sequence number of the last validly            received packet.                                        16

3. Wait.                 The Wait Control Sequence informs the receiving entity that            the sending entity requires additional time to process what            it has received.  Wait is sent as:                     <DLE> ;                   4. Negative Acknowledge.                 When the receiving entity receives a packet with an            unexpected <Sequence>, improper checksum or times out, it            sends a Negative Acknowledge to the other entity.  This            consists of the single byte:                      <NAK>  0x15                 When the sending entity receives a NAK, it initiates a            Resynchronization Process by sending two <ENQ> characters            and waiting until it receives two identical Positive            Acknowlegements.              5. Panic Abort.                 The B Plus Protocol recognizes a series of four <DLE>            characters as a panic abort.  The need for such a facility            can arise if the Responder computer becomes locked up during            a transfer, and it is necessary to reboot.  Since <DLE> is            really control-P, sending it is a very simple procedure.                 In addition, the Initiator recognizes the seqeunce:                      <DLE> +                 while waiting for a Positive Acknowledge as being incorrect,            and will abort the transfer.  This can happen as above, but            before the operator can press control-P the Initiator sends            an <ENQ> during a down-load to find out what is happening.            The Responder, now in terminal emulation mode, responds            with:                      <DLE> + + <DLE> 0                 and the Initiator will abort cleanly.  For this reason it is            essential that any B Plus implementation observe the rules            for <ENQ>:  If in terminal mode, send <DLE> + + <DLE> 0;            otherwise send <DLE> Sequence.                                             17

E. Negotiation of Transport Parameters.              The Initiator begins a B Plus protocol session by sending:                        <ENQ>              while the Responder is in terminal mode.  The Responder sets         itself up for:                  o  Standard Checksum             o  Default Quoting             o  512 byte data block             o  Sets its Sequence to 0x30 (`0')             o  transmits the sequence:                            <DLE> + + <DLE> 0                  o  returns to terminal mode.              The Initiator, detecting the `<DLE> + +', sends a Transport         Parameters Packet using Standard Checksum and quoting of         0x00 thru 0x1f and 0x80 thru 0x9f.  When the Responder         properly receives this packet, it must transmit a Transport         Parameters Packet indicating its own parameter settings.         Once the Initiator Acknowledges the Responder`s Transport         Parameters Packet, each entity then establishes its         operating configuration based on the minimal matching         parameters.  In particular, the Initiator and Responder each         choose the minimum of:                 Own       Other's         Parameter   Parameter                 WS           WR    <- Note the cross-comparison of            WR           WS    <- these parameters.            BS           BS            CM           CM            TL           TL            DR           DR            UR           UR            FI           FI              and the inclusive-or of the Quoting Sets.              For example, if the Initiator sends the sequence:                       0x01 0x01 0x08 0x01 0x01 0x00                  0x14 0xd4 0x00 0x00          <- Quoting Set                  0x00 0x00 0x00 0x00          <-                  0x01 0x00 0x00               <- DR UR FI              and the Responder returns:                       0x00 0x01 0x04 0x00 0x01 0x00                                        18

0x54 0xd4 0x00 0x00          <- Quoting Set                  0x40 0x50 0x00 0x00          <-              The following parameters are used by both entities:                 Parameter  Initiator  Responder               WS         0x01      0x00      Initiator sends 1                                              packet ahead.                    WR         0x00      0x01      Responder sends 1                                              packet and waits for                                              Postive Acknowledge               BS         0x04      0x04      Both use 512 byte                                              data body               CM         0x00      0x00      Standard Checksum                    The following characters will be quoted when               transmitted by either entity:                       0x01 0x03 0x05 0x10 0x11 0x13 0x15 0x81 0x91 0x93                  SOH  ETX  ENQ  DLE  XON  XOFF NAK  SOH  XON  XOFF                    This would be the quoting used for a Responder               having a modem which treats the 0x01 as "Escape to               command mode" and in addition responds to flow               control characters (0x11 0x13 0x91 0x93).              Since the Responder did not transmit the DR, UR, and FI         parameters, the initiator sets them to 0x00.              The next packet transmitted will be expected to obey the         negotiated Transport Parameters.                                                     19

F. Terminal Program States.              This section describes the expected state transitions for a         terminal program which supports the B Plus Protocol.              Current State      Event     Action         Next State         ------------------------------------------------------------         Terminal      <ENQ> Rcvd.    Return:        Terminal                                  <DLE> + + <DLE> 0                                  Set up for normal                                  B Protocol                            <DLE> Rcvd.    None.          DLE_Seen                            <Other>        Process as an                                      ASCII char.    Terminal              ------------------------------------------------------------         DLE_Seen      <B> Rcvd.      None.          Get_First_Packet                            <other> Rcvd.  None.          Terminal              ------------------------------------------------------------         Get_First_Packet        Receive the Packet                       Valid packet :                            <+>      Process Transport     Terminal                                 Parameters; set                                 up for B Plus if                                 successful exchange                            <T>      Invoke Transfer       Terminal                                 Process                            <other>  Return Failure        Terminal                                 Packet N              ------------------------------------------------------------                                             20

G. Wait for Acknowledge.              The Wait for Acknowledge procedure is the work-horse of the         B+ Protocol.  It is implemented as a Finite State Automaton         having thirteen states as follows:                State               Description         ----------------   ----------------------------------------         S_Get_DLE          Wait for <DLE> from Responder.         S_DLE_Seen         Get character following the <DLE>.         S_DLE_B_Seen       Receive Packet Sequence Number.         S_Get_Data         Receive data portion of packet.         S_Get_Check        Receive check value.         S_Get_CRC          Receive CRC-16.         S_Verify_CRC       Compare received to calculated CRC-16.         S_Verify_CKS       Compare received to calculated checksum.         S_Verify_Packet    Check for failure packet; verify packet                            sequence number.         S_Send_NAK         Count error; send <NAK> to Responder.         S_Send_ACK         Send ACK sequence to Responder.         S_Send_ENQs        Count error; Send two <ENQ> characters to                            Responder.         S_Resend_Packets   Retransmit packet(s) to Responder.                   The State Transition table follows:              State              Event          Action              Next State         ----------------   ------------   -----------------   -----------         S_Get_DLE          <DLE>           ...                S_DLE_Seen                            <NAK>           ...                S_Send_ENQ                            <ENQ>           ...                S_Send_ACK                            <ETX>           ...                S_Send_NAK                            timeout         ...                S_Send_ENQ                            other           ...                S_Get_DLE              S_DLE_Seen         <digit>        Release Packet      return success                                               -or-                                           If <ENQ> sent       S_Resend_Packets                                           else                S_Get_DLE                            <B>             ...                S_DLE_B_Seen                            <;>             ...                S_Get_DLE                            <ENQ>           ...                S_Send_ACK                            timeout         ...                S_Send_ENQ                            other           ...                S_Get_DLE              S_DLE_B_Seen       <ENQ>           ...                S_Send_ACK                            <char>         Save <char>                                           Intit Check Value   S_Get_Data                            timeout         ...                S_Send_NAK                                             21

S_Get_Data         <ETX>           ...                S_Get_Check                            timeout         ...                S_Send_NAK                            <ENQ>           ...                S_Send_ACK                            <char>         Add <char> to                                             check value                                           Store <char> in                                             buffer            S_Get_Data              S_Get_Check        <char>         If Check Method is                                             CRC_16            S_Get_CRC                                             Checksum          S_Verify_CKS                            timeout         ...                S_Send_NAK              S_Get_CRC          <char>         Add <char> to                                             Received CRC-16   S_Verify_CRC                            timeout         ...                S_Send_NAK              S_Verify_CRC        ...           If Received CRC-16                                             = Calc. CRC-16    S_Verify_Packet                                           else                S_Send_NAK              S_Verify_CKS        ...           If Received CKS                                             = Calc. CKS       S_Verify_Packet                                           else                S_Send_NAK              S_Verify_Packet    ...            If Saved <char>                                             = expected Seq                                                number         Return size                                              -or-                                           If Packet Type                                             = `F'             Return size                                           If duplicate                                             packet            S_Send_ACK                                           else                S_Send_NAK                S_Send_NAK         ...            Count Receive                                              Error                                           If too many errors  Return failure                                           If not Aborting                                           then Send <NAK>                                                               S_Get_DLE              S_Send_ACK         ...            IF not Aborting                                           then Send ACK sequence                                                               S_Get_DLE              S_Send_ENQ         ...            Send <ENQ><ENQ>     S_Get_DLE              S_Resend_Packets   ...            Send all un-ACKed                                             packets.          S_Get_DLE              Note:  "Aborting" means that a Failure Packet has been sent.  If                Failure Packet has been sent, all outstanding packets are                assumed to be ACKed.                                        22

H. Packet Send Ahead              One of the most useful features of the B Plus Protocol, from         the user's point of view, is its ability to send more than         one packet ahead of receiving the acknowledgment of the         oldest one.  This improves thru-put by overlapping the I/O         performed on both the Initiator and Responder computers,         including the time required for the Responder computer's         acknowledgement to arrive at the Initiator and wake up the         Initiator's application program.              The Packet Send Ahead is accomplished by keeping a set of         buffers in memory, one for each permitted outstanding         packet.  Each buffer contains the corresponding packet's         <Sequence>, count, and the data itself.  In addition a         counter of outstanding packets (ie, not yet ACKed) is         maintained and incremented whenever a packet is sent for the         first time.              Two indicies into the set of buffers are used to keep track         of the Send Ahead status.  One keeps track of which buffer         holds the oldest packet and the other which buffer may be         filled with new data.              When a packet is about to be sent, the outstanding count is         checked to see if the maximum number of outstanding packets         has been reached.  If so, a routine (GetACK) is invoked         which waits for the ACK to arrive for the oldest outstanding         packet.  Several possibilities exist:             o The oldest packet is ACKed.  In this case, move the               Oldest Outstanding index to point to the next oldest               packet and decrement the outstanding packet count.             o Some other packet in the buffer pool has been ACKed.               This implies an ACK has been missed.  Move the Oldest               Outstanding index to point to the buffer following the               received ACK, and decrement the Outstanding Count               accoringly.             o A NAK is received.  Retransmit all packets which are               outstanding.              If the Outstanding count is still at the maximum, stay in         GetACK.              Finally, a procedure is necessary which calls GetACK until         either a failure occurs or the Outstanding count is zero.                                             23

The Send Ahead Process proceeds as follows:                                                     Receiver's         Packet   Sequence  Buffer    Outstanding    Response           1        `1'       0           1            ...           2        `2'       1           2            ...           3  GetACK                                 ACK `1'                    `3'       0           2            ...           4  GetACK                                 ACK `2'                    `4'       1           2            ...           5  GetACK                                 ACK `3'                    `5'       0           2            ...   (Last)              GetACK                                 ACK `4'                                          1            ...              GetACK                                 ACK `5'                                             24

I. Implementation Considerations              1. Time-Out                 A per-character time-out of ten seconds should be used.            Using anything less can result in a loss of good control            over the exchange of packet and control sequences.  If            conditions over a switched network degrade sufficiently, a            small time out can lead to excessive retransmissions of            data.  On the other hand, longer time-out values will result            in a more sluggish detection of protocol breakdown.                 The receiving entity should be silent when a timeout occurs.            That is, a <NAK> should not be sent when a timeout occurs            while receiving a packet.  The reason is that under severely            degraded network loading it may happen that it takes longer            for the sending entity's data to arrive than the receiving            entity's timeout interval.  This can lead to many kinds of            situations in which there is excessive retransmission of            data.  The question to be answered by the implementor is:            "Am I more concerned about how fast I can shut down the            transfer when the sending entity goes away than I am about            getting the transfer to occur regardless of how long it            takes?"  If the answer is that response to a vanished sender            is more important, then send a <NAK> when a timeout occurs.            If getting the job done is more important, then don't send            the <NAK> on timeout.              2. Packet Size.                 The following chart shows the percentage of houskeeping overhead            for various packet sizes (not including quoted characters):                                                           Packet               Data  Packet                      Transmission Time               Size   Size    Useful    Overhead   300 1200 2400               ----  ------   ------    --------  ---- ---- ----               128    135     94.8 %    5.2 %      4.5  1.1  0.6               256    263     97.3 %    2.7 %      8.8  2.2  1.1               384    391     97.7 %    2.3 %     13.0  3.3  1.6               512    519     98.7 %    1.3 %     17.3  4.3  2.2               640    647     98.9 %    1.1 %     21.6  5.4  2,7               768    775     99.1 %    0.9 %     25.8  6.5  3,2               896    903     99.2 %    0.8 %     30.1  7.5  3.8              1024   1031     99.3 %    0.7 %     34.4  8.6  4.3                 At first glance it might seem reasonable to always use 1024 byte            data packets.  However, consideration should be given to the            amount of time required to recover from a transmission error.            For example, using 1024 byte data packets at 300 baud, over a            minute of excessive transmission time would result (two packets)            each time an error occured.  Experience shows that keeping the            packet transmission time to between four and five seconds            results in a reasonably comfortable recovery time.  So, for best                                        25

over-all performance, the following packet sizes are            recommended:                    Baud      Size               ----      ----                300       128               1200       512               2400      1024              3. Controlling Excessive Retransmissions.                 Over a very noisy link the send-ahead nature of B Plus might            cause the performance to degrade due to retransmission of            packets.  To circumvent this degradation, the following            heuristic is recommended:                    Initialize  SA_Error_Count to zero.               If packets are retransmitted, increment SA_Error_Count by 3.               If SA_Error_Count is greater than or equal to 12, drop out of                    Send-Ahead mode (ie, revert to send and wait mode).               When a packet is ACKed, and SA_Error_Count is greater than                    zero,  decrement SA_Error_Count by one.                 The values (increment by 3, limit of 12, decrement by 1) can be            varied in accord with the implementor's experience, perhaps            being established by user-definable parameters.                                                  26

J. B Plus Transport Layer.              Some distributed applications (that is, applications in         which the Initiator and Responder each perform some of the         work) require that blocks of information be exchanged.         Running such applications over normal communication channels         can cause loss or altering of data, yielding unpredictable         results.  To aleviate this possibility, the B Plus Protocol         provides the ability to use the underlying packet structure         to exchange blocks of data with error detection and         retransmission.              Entering the Transport Layer operation is more complicated         than the File Transfer mode previously described:                  o  The user invokes a CompuServe application             o  The application program transmits a CompuServe                Application Protocol escape sequence.             o  The user's terminal program responds with an escape                sequence which informs the host application that it                must use the Transport Layer.             o  The host program initializes its B Plus routines,                setting the Transport Layer parameter to 0x01.             o  The terminal program waits until its own B Plus                     routines have been initialized, also setting the                     Transport Layer parameter to 0x01.             o  Once the host and remote B Plus routines have                     exchanged their Transport Layer Parameters, the                Transport Layer is in effect.  The succeeding action                is a function of the application.              From an implementation standpoint, the primary differences         from the B Plus Protocol so far described are:                  o  Separate Packet Sequence Numbers are maintained for                sending and receiving.             o  Received Packets are placed in a first-in-first-out                queue.             o  Another level of software is placed between the                     packet handlers and the application.  This level is                responsible for sending and receiving blocks of data,                packaging them into protocol-sized packets.              The following Packet Types are used in the Transport Layer:                  `M':  This is a data packet; another data packet follows.                  `L':  This is the last data packet for the current block.              A Failure Packet will terminate the transport layer for both         entities.                                             27

K. Initiation of a B Plus Session.              The following state machine describes the process of a host         entity initiating a B Plus Session.  Its starting state is         Send_ENQ.              State         Received  Action                   Next State         ------        --------  ----------------------   ----------         Send_ENQ        ...     Transmit <ENQ>                                 Set Standard Checksum                                 Set 512 byte Data Block                                 Set default quote set                                 Clear Plus_Seen and                                 Plus_Plus flags          Get_DLE         -----------------------------------------------------------         Get_DLE       <DLE>     ...                      Get_Digit                       <+>       If Plus_Seen true,                                   then set Plus_Plus                                        ...               Get_DLE                       timeout                            Send_ENQ         ------------------------------------------------------------         Get_Digit     <digit>   Initialize Send and                                   Receive Sequence                                   Numbers                Send_Parameters                       <+>       Set Plus_Seen            Get_DLE                       timeout       ...                  Send_ENQ            ------------------------------------------------------------         Send_Parameters ...     If Plus_Seen true,                                   then                                      if Plus_Plus true,                                        then set to quote                                           0x00-0x1f and                                           0x80-0x9f                                      Send Parameters Packet                                      Receive Parameters Packet                                      Establish session's parameters                                                                                                                     Exit         -------------------------------------------------------------                                                  28

L. Supporting the Various B Protocol Versions              There are three versions of B Protocl which an implementor must         be aware of.  Any new implementation should follow the B Plus         Protocol as described in this document to ensure full         compatibility with all existing CompuServe Host software.  The         three versions are:                       o  "Classic" B Protocol                  o  An interim version of B Plus (called Quick B)                  o  B Plus as described in this document              The differences are:                  Initial <ENQ> response:                  Classic B:      <DLE> digit                  QuickB:         <DLE> + <DLE> digit                  B Plus:         <DLE> + + <DLE> digit                  Default Quoting Sets:                  Classic B:      0x00 -> 0x1f                  QuickB:         0x00 0x03 0x05 0x10 0x11 0x13 0x15                  B Plus:         0x03 0x05 0x10 0x11 0x13 0x15                  Extended Quoting:                  Classic B:      none                  QuickB:         Specified by the DQ Parameter:                                  0x00 : 0x00 0x03 0x05 0x10 0x11 0x13 0x15                                  0x01 : 0x03 0x05 0x10 0x11 0x13 0x15                                  0x02 : 0s03 0x05 0x10 0x11 0x13 0x15 0x91 0x93                                  0x03 : 0x00-0x1f and 0x80-0x9f                  B Plus:         Established by the Q1 thru Q8 parameters.                                  (Ignore R0)                  Quoting of the Paramters Packet:                  Classic B:      No paramaters packet                  QuickB:         0x00 -> 0x1f                  B Plus:         0x00 -> 0x1f and 0x80 -> 0x9f                  Response to a <NAK>:                  Classic B:      Retransmit the last packet                  QuickB:         Resynchronize with <ENQ><ENQ>, wait for                                  two identical ACK sequences, retransmit                                  as necessary.                  B Plus:         Same as QuickB.                                             29

CompuServe Host software will support all three B Protocol         versions.  In order to do so, it relies on the initial response         to <ENQ> to determine what version is being used in the terminal         software.  The defaults given in Section D.1 ("Enquire") are         intended to reflect "Classic" B Protocol.  If the Host receives                          <DLE> <+> <DLE> <digit>              it sends its Transport Parameters Packet using "Classic" B data         quoting (0x00 -> 0x1f).  If two "+" chracters are received, it         will quote 0x00 -> 0x1f and 0x80 -> 0x9f.  This is done to         maximize the chances of getting the packet through most modems         and networks.  Once the Transport Parameters Packets have been         successuflly exchanged, the Host software uses the negotiated         quoting set, and the remote is expected to do likewise.              Negotiation of quoted characters allows implementors to provide         configuration set up to cope with a wide variety of modems,         oeprating system, network, and other situations which can         adversely affect thr transmission of 8-bit binary data.                                                  30

M. Check Value Calculation.              1. Standard B Protocol Checksum.                 The following C-language code fragment illustrates the            Standard Checksum calculation:                 int              checksum;                 do_checksum (ch)            unsigned int ch;            {                checksum = checksum << 1;                     if checksum > 0xff                    checksum = (checksum  & 0xff) + 1;                     checksum += ch & 0xff;                     if checksum > 0xff                    checksum = (checksum & 0xff) + 1;            };                                             31

2. XMODEM-Style CRC-16.                 /*             * Calculates XMODEM-style CRC (uses the CCITT V.41             * polynomial but             * completely backwards from the normal bit ordering).             */            static unsigned crc_xmodem_tab[] = {            0x0000,0x1021,0x2042,0x3063,0x4084,0x50A5,0x60C6,0x70E7,            0x8108,0x9129,0xA14A,0xB16B,0xC18C,0xD1AD,0xE1CE,0xF1EF,            0x1231,0x0210,0x3273,0x2252,0x52B5,0x4294,0x72F7,0x62D6,            0x9339,0x8318,0xB37B,0xA35A,0xD3BD,0xC39C,0xF3FF,0xE3DE,            0x2462,0x3443,0x0420,0x1401,0x64E6,0x74C7,0x44A4,0x5485,            0xA56A,0xB54B,0x8528,0x9509,0xE5EE,0xF5CF,0xC5AC,0xD58D,            0x3653,0x2672,0x1611,0x0630,0x76D7,0x66F6,0x5695,0x46B4,            0xB75B,0xA77A,0x9719,0x8738,0xF7DF,0xE7FE,0xD79D,0xC7BC,            0x48C4,0x58E5,0x6886,0x78A7,0x0840,0x1861,0x2802,0x3823,            0xC9CC,0xD9ED,0xE98E,0xF9AF,0x8948,0x9969,0xA90A,0xB92B,            0x5AF5,0x4AD4,0x7AB7,0x6A96,0x1A71,0x0A50,0x3A33,0x2A12,            0xDBFD,0xCBDC,0xFBBF,0xEB9E,0x9B79,0x8B58,0xBB3B,0xAB1A,            0x6CA6,0x7C87,0x4CE4,0x5CC5,0x2C22,0x3C03,0x0C60,0x1C41,            0xEDAE,0xFD8F,0xCDEC,0xDDCD,0xAD2A,0xBD0B,0x8D68,0x9D49,            0x7E97,0x6EB6,0x5ED5,0x4EF4,0x3E13,0x2E32,0x1E51,0x0E70,            0xFF9F,0xEFBE,0xDFDD,0xCFFC,0xBF1B,0xAF3A,0x9F59,0x8F78,            0x9188,0x81A9,0xB1CA,0xA1EB,0xD10C,0xC12D,0xF14E,0xE16F,            0x1080,0x00A1,0x30C2,0x20E3,0x5004,0x4025,0x7046,0x6067,            0x83B9,0x9398,0xA3FB,0xB3DA,0xC33D,0xD31C,0xE37F,0xF35E,            0x02B1,0x1290,0x22F3,0x32D2,0x4235,0x5214,0x6277,0x7256,            0xB5EA,0xA5CB,0x95A8,0x8589,0xF56E,0xE54F,0xD52C,0xC50D,            0x34E2,0x24C3,0x14A0,0x0481,0x7466,0x6447,0x5424,0x4405,            0xA7DB,0xB7FA,0x8799,0x97B8,0xE75F,0xF77E,0xC71D,0xD73C,            0x26D3,0x36F2,0x0691,0x16B0,0x6657,0x7676,0x4615,0x5634,            0xD94C,0xC96D,0xF90E,0xE92F,0x99C8,0x89E9,0xB98A,0xA9AB,            0x5844,0x4865,0x7806,0x6827,0x18C0,0x08E1,0x3882,0x28A3,            0xCB7D,0xDB5C,0xEB3F,0xFB1E,0x8BF9,0x9BD8,0xABBB,0xBB9A,            0x4A75,0x5A54,0x6A37,0x7A16,0x0AF1,0x1AD0,0x2AB3,0x3A92,            0xFD2E,0xED0F,0xDD6C,0xCD4D,0xBDAA,0xAD8B,0x9DE8,0x8DC9,            0x7C26,0x6C07,0x5C64,0x4C45,0x3CA2,0x2C83,0x1CE0,0x0CC1,            0xEF1F,0xFF3E,0xCF5D,0xDF7C,0xAF9B,0xBFBA,0x8FD9,0x9FF8,            0x6E17,0x7E36,0x4E55,0x5E74,0x2E93,0x3EB2,0x0ED1,0x1EF0};                 unsigned CRC;            UpdCrc (Byte)            unsigned int ByteC;            {  CRC =                 crc_xmodem_tab [(CRC >> 8 ^ Byte) & 0xff] ^ (crc << 8);            };                                             32

N. Interrogation.              Although not part of the B Plus Protocol, implementors are urged         to support the CompuServe Interrogation Sequence.  This process         passes information to the CompuServe Host to identify the         various options supported by the terminal program.  Host         software will generally perform an Enquire (Section D.1) first         to determine if the terminal program supports B Protocol.  If B         Protocl ("Classic," QuickB, or B Plus) is supported, the host         then sends the sequence:                       <ESC> I              When the terminal program receives this sequence of characters,         it transmits an Interrogation Response as follows:                  <#> <Mod> <Ver> <F-codes> <+> <Check> <CR>              where:                  <#>          Identifies the string as an Interrogation                          Response                  <Mod>        Specifies the specific hardware or operating                          system, or a generic class of software.  It is                          always three characters in length.  In addition                          to several reserved codes, the following                          generic codes are defined:                                APX - Apple (TM) II/Macintosh                                ATX - Atari (TM) 8, 16, and 32 bit                                AUT - Auto-Script Program                                CCX - Commodore (TM) C64 and Amiga                                CPM - CP/M (TM) Operating System                                IBX - IBM (TM) Compatible                                XXX - Other model/type                  <Ver>        Gives the terminal software version number.  It                          may contain Letters, Digits, and Periods.                  <F-codes>    A series of character fields giving the                          options supported.  Each <F-code> is followed                          by a comma.  The currently recognized <F-codes>                          are as follows:                                 Cursor/Screen Control                                 AC : ANSI Color                                 CA : ANSI/VT100 Cursor Control                                 CC : VIDTEX/VT52 Cursor Control                                 CW : Wide mode (Double width characters                                           invoked by <ESC> <m>                                           uninvoked by <ESC> <l> (lower                                                case L)                                 SSyx : Screen Size; y gives number of                                        lines + 31, x gives number of                                        33

columns plus 31.  SS7o identifies                                        a 24 line by 80 column screen.                                        It is suggested that the SS field                                        always be present.                                                        Graphics                                 GF : GIF graphics                                 GH : High resolution RLE graphics                                 GM : Medium resolution RLE graphics                                 NF : Full NAPLPS graphics                                 Protocols                                 AP : Recognizes and properly responds to                                      the CompuServe Application Protocol                                      escape sequences                                 CB : Capture Buffer                                 PB : B Protocol (includes QuickB and B                                      Plus)                                 PX : XMODEM                                 Miscellaneous                                 HC : Hard Copy (printer connected)                                 XX : Ignored                            All other <F-codes> are reserved for future use.                  <Check>      This is a check value derived by adding                          together all characters in the Interrogate                          Response up to and including the +,                          truncating the sum to 16 bits, and converting                          it to an ASCII digit string.                  <CR>         This is the ASCII Carriage Return and serves to                          terminate the Interrogate Response.              If the <Check> value does not match, the host will send another         <ESC> I and compare the string it receives to the previous one.         If the two strings match, it is accepted, otherwise the <ESC> I         and matching will continue for ten trials before giving up.                                             34

A sample Interrogate Response is:                  #IBX,CC,GH,GM,PB,+1035<CR>              which specifies:                  o IBX : An IBM (TM) compatible system             o CC  : Supports standard VIDTEX/VT52 cursor controls             o GH  : Supports high-resolution RLE graphics             o GM  : Supports medium-resulution RLE graphics             o PB  : Supports B Protocol             o 1035: Sum of "#IBM,CC,GH,GM,PB,+"              For additional information concerning the Interrogation         Response, please contact CompuServe Incorporated, Microcomputer         Software Development.                                             35