The "C1" Protocol-----------------The "C1" description is exceedingly long, so it has been broken up into 11more easily digestible sections. If you haven't read any of this before,please read them ALL, in order.Topic                            Section-----                            -------Inception & Concepts ........... C1-1A Simple Conversation .......... C1-2Communication Codes & Checksums. C1-3Statement and Listen Loops ..... C1-4Synchonization Lock ............ C1-5Block Structure ................ C1-6Varying Block Size ............. C1-7Communication Syntax ........... C1-8Syntax Description ............. C1-9The "Endoff" Situation ......... C1-10Transfering File Type .......... C1-11Section C1-1------------Inception---------During the summer of 1981, when I first got the idea of putting up a BBS, Istarted work on a simple protocol for transfering programs to and from theBBS. This protocol was similar in structure to XMODEM, and had about the samereliability. Under good line conditions, it would give error free transfers(this was to be expected). Under moderate noise conditions, the protocol wouldhold up, and would still give error free transmissions. It was under poor lineconditions that it, and XMODEM, would fall apart.In the summer of 1984, I started work on a very ambitious project; to producea protocol that was both fast, and extremely reliable, even under the worst ofline conditions. From this work came the "C1" protocol; not a simpleblock/checksum affair, but a complete communication system for the computer.Be warned, therefore, that understanding the ins and outs of "C1" will not beeasy, but with enough patience, there's no reason why even the least skilledprogrammer cannot be comfortable with it.Concepts--------The concept behind the "C1" protocol was simple; to allow two computers to"talk" with one another (while transferring data) in such a way that nothingshort of a complete distortion of the transmission line could result in amisunderstanding. If this concept could be realized, then files could betransferred between computers without fear of line noise causing a breakdownin the protocol, or that the received data would differ, in any way, from thatwhich was sent. Nothing is perfect though, and I don't, for a minute, claimthat "C1" is completely infallible, but I can say, with reasonable comfort,that "C1" can deliver bad line accuracy not found in any other microcomputertransfer protocols. For this accuracy though, there is a price to pay, and itis complexity; the protocol is extremely difficult to duplicate without acomplete and utter understanding of the intricate workings of "C1". Thisdocument will attempt to give you that required understanding.Section C1-2------------A Simple Conversation---------------------In first deciding how the protocol would function, I thought of how two peoplecould carry on a conversation under high noise conditions, wheremisunderstanding would be the norm. The senario I'm going to give differs fromthe protocol in that the people talking have no way of verifying the accuracyof what they believe they have heard. What it is meant to demonstrate is howthe the two computers "talk" with one another, and discuss the neccessaryrepetition, or non-repetition, of each block of data (the cornerstone of achecksum based transfer protocol).Ken and John are attempting to assemble a machine in the middle of a verynoisy machine shop. Ken reads the instructions to John, who carries them out.Even at close proximity, the two have difficulty hearing one another, so theyadopt of form banter which allows each instuction to be verified andacknowledged. Here is how the conversation might go:John: Put part "A" in hole "D".Ken: Understood, putting part "A" in hole "D".John: Acknowledged, let me know when you are ready for the next instruction.Ken: Go ahead, what do I do next?John: Put screw "E" through slot "T".Ken: I didn't understand that, could you please repeat.John: Oh, ok, tell me when you're ready for that instruction again.Ken: Ready now.The conversation continues on in this fashion, guaranteeing that both John andKen are fully aware of what the other is doing. In real life, people wouldn'thave the patience to keep up that sort of banter, but that's why they makemore mistakes than a computer. It is just this sort of "conversation" that thetwo computers have between each other, only the language is different; theinstruction is replaced by the block of data, and all other statements byspecial codes.Section C1-3------------Communication Codes-------------------One of the areas where simple protocols fall apart is in the transmission of"handshaking codes". It's called handshaking because is implies that the twocomputers are having a dialogue, rather than a monologue. These otherprotocols rely on single byte (8 bit) words for their communication codes, andthat could spell trouble, since the likelihood of any one 8 bit code beingtransposed into another is greater than for multiple byte codes. For thisreason, "C1" uses 3 byte (24 bit) codes which are sufficiently different thatthe likelihood of a transposition is extremely low. Not only that, but as youwill soon learn, the method of receiving 3 byte codes is designed such that ifthere is sufficient line noise to make the neccessary transpositions, therewould most likely be extra characters sent; "C1" can avoid this situation.Five distinct codes are used in the protocol; "GOO", "BAD", "ACK", "S/B", and"SYN". Each has it's own meaning, just like any English word, and all are usedin a specific sequence such that synchronization difficulties would beautomatically identified and corrected.Checksums---------When a block of data is sent, we must have a way of determining if it iscorrectly received or not. This is accomplished by using what is known as achecksum. Quite simply, a checksum is a number which is mathematically derivedfrom all the bytes within the block. The receiving computer recalculates thesum and compares it with the sum it received along with the block.Theoretically, any fault in the transmitted data will result in the twochecksums not matching; but that's theory. In reality, the accuracy of thechecksum is based on the type of mathematical operation used to calculate it,and what kind of noise it encounters.The simplest way to create a checksum is to add up all the ASCII values of thebytes contained in the block. This is fine for many types of errors, but notthe type which inverts a particular bit. Should two identical inversions occuron two opposite bits, the sum will remain the same. For example, take thefollowing two bytes:     11010011 = 211Plus 01101101 = 109     --------   ---                320Now assume that the forth bit from the right of both of these bytes becomesinverted by line noise:     11011011 = 219Plus 01100101 = 101     --------   ---                320As you can see, the sum remains 320, even though line noise has made obviouschanges to the bytes. A better system is one called "Cyclic Redundancy", whichworks on a somewhat different principle. The checksum is 16 bits long, and iscreated in the following fashion; each byte from the block is Exclusive OR'edwith the low order part of the checksum. The checksum is then ROTATED one bitto the left, and the procedure repeated with the next byte.  Even this highlysuperior method can be tripped up, so I have combined BOTH an additivechecksum and Cyclic Redundancy checksum to create one very hard to beat 32 bit"super" checksum.Section C1-4------------Listening For Code Words------------------------Although 3 byte code words are more reliable than 1 byte code words, nothingis perfect. It was once said that if you let an infinite number of monkeysbash away at typewriters for an infinite amount of time, one of them wouldeventually type "To be or not to be, that is the question". Although thisstretches statistical probability to it's limit, this kind of thing can easilyhappen on a smaller scale; the letters "GOO" could quite conceivably beproduced by purely random line noise.To try and eliminate ALL possible errors isn't feasible, but "C1" makes anattempt at trying to eliminate as many as possible. One reasonably probablefact is that any noise capable of randomly producing "GOO", would not stopthere; more likely, it would produce a string of characters, something like"HGOOEK". Were we to allow the protocol to listen exclusively for three lettercombinations, it would most assuredly pick out the "GOO" in that string.My specifications for "C1" call for a code recognition routine which will ONLYmake code word comparisons on the LAST 3 RECEIVED bytes. This is accomplishedin my coding by going back and testing for further characters after I haveidentified a three byte code word. Should another byte be present, theidentified code word is thrown away, and the search will continue.Statement and Listen Loops--------------------------One immediate drawback to the system described above is that a REAL code word,masked within some random noise, would be rejected by the receiving computer.This would also be true of a code word simply damaged by noise (like "GOE").For a protocol to be impervious to this sort of corruption, it must be capableof restating code words over and over until the receiving computer canunderstand, yet it must also have a way of knowing whether the receivingcomputer got the code word or not. This was a fact that eluded me when I wrotethe original protocol. When we talk to other people, the cornerstone ofunderstanding is recognition. If we ask "What do you think?", yet get noreply, we ask again. Only when we receive a reply from the person to whom weare talking

do we continue on with our next statement. It would be pointlesswasting our breath on someone who isn't listening.Within "C1", communication between computers is handled through a similarsystem which I call the "Statement and Listen Loop". It's quite simple really;when one computer has to "say" something to the other, it does so, then waitsfor a predetermined time for a known response. Should it fail to receive aresponse within that period of time, the code word is said again, and thecomputer listens for the reply. This continues until the required response isheard. The system is further enhanced by the fact that both computers areALWAYS engaged in a "Statement and Listen Loop".Section C1-5------------Synchronization Lock--------------------That rather ominous sounding title is actually rather simple; it refers to acondition whereby the "Statement and Listen Loops" of each computer becomelocked together. This is analogous to two people speaking at the same time,over and over, such that no effective communication takes place. In order toguarantee that the two computers never get into this state, the wait times ofthe loops are altered slightly.Assume that the fixed wait loop time was 0.5 seconds; this is called a "Short"wait. We also have a "Long" wait, which would be slightly longer, say 0.6seconds (actually, the delay within a "Statement and Listen Loop" is notparticually critical, but should be somewhere in the neighbourhood of one halfsecond). Each time the computer goes through an SLL, a counter would determinewhich type of wait to use; Long or Short. The sequence is broken into three;the transmitting computer will use a Long-Long-Short, while the receivingcomputer will use a Short-Short-Long.Section C1-6------------Block Structure---------------Each block of data contains somewhat more than just a collection of characterstaken from disk, it also contains a "header". The header is 7 bytes long, andcontains the following information:Byte 1: Low part of ADDITIVE checksumByte 2: High part of ADDITIVE checksumByte 3: Low part of CLC checksumByte 4: High part of CLC checksumByte 5: Size of NEXT blockByte 6: Low part of Block NumberByte 7: High part of Block NumberAs you remember from the section on "checksums", there are two distinctlydifferent, 16 bit (2 byte) checksums. One is an additive checksum, composed ofthe mathematical sum of the ASCII values of all the DATA bytes (and bytes 5through 7 of the header). The other checksum is calculated using Cyclic (CLC)Redundancy (on the same bytes). These 32 checksum bits are placed in the first4 bytes of the header.The 5th byte is the length of the NEXT block. This may seem odd to some, butconsider the difficulties in sending the size of the current block in thatself same block. You need to know the block size to calculate the checksum,but you can't know for sure that the block size is correct unless you haveverified the checksum. We call this a Catch-22. By sending the size of anygiven block in the PREVIOUS block, the size is known for a fact BEFORE thechecksum is calculated.In the 6th and 7th byte are the block number. This was added quite early on inthe development of "C1" under the assumption that it would be necessary (as itis in XMODEM). As it turned out, "C1" uses a method of handshaking which makesthis unnecessary. None the less, my specifications call for it's inclusion, ascertain uses of the block number could be made. Also, the high order part ofthe block number (byte 7 of the header) is used to flag the last block.Section C1-7------------Varying Block Size------------------The reason that block size was included in the header was originally to allowthe last block only to vary in size (one can never guarantee that the amountof data to be sent will divide nicely into a preset block size). It quicklydawned on me that "C1" was set up in such a way that ANY block size could beused for ANY block in the transmission. Varying block size has it'sadvantages; under reasonably clean line conditions, large blocks transmit themost data with the least handshaking (which is mildly time consuming). Smallerblocks are superior under bad noise conditions, since smaller blocks run ahigher chance of making it through the noise unscathed; and should it stillfail to make it, less time is required to repeat a smaller block.My current implementation of "C1" allows the user to pick a fixed block sizebetween 40 and 255 bytes, but in other implementations, there is no reason whyblock size couldn't be varied DURING transmission to adapt to CHANGING lineconditions.One final thing concerning block structure is how would one presume to knowthe size of the FIRST BLOCK if that is revealed only in the block that camebefore it (quite a paradox). "C1" requires that the first block contain ONLY aheader, which would make that block 7 bytes long. This header would do littlemore than supply the receiving computer with the size of first REAL block.Accuracy of this first "dummy" block is guaranteed since it must still passthe checksum tests. You must make the block number for this dummy block "0".Section C1-8------------Communication Syntax--------------------Now that you understand block structure, handshaking methods, and code wordvocabulary, it comes time to find out how this all comes together. Mostprocotols have very simple handshaking between blocks which is easy to tripup, given sufficiently noisy conditions. Usually, the transmitting computersends the block, then waits for a response from the receiving computer; either"good" or "bad". The transmitting computer then proceeds to send the nextblock (if "good") or resend the last block (if "bad"). This system falls apartthe moment the transmitting computer receives a false indication of "good" or"bad" and goes on to transmit the wrong block (and whether the receivingcomputer likes it or not, it has to tackle with another block). Should thingsget out of sync, and the transmitting computer sends the next block when itshould have sent the last one again, XMODEM attempts to make corrections byuse of the block number encoded within each block."C1" does nothing so crude; it's very communication syntax guarantees thatneither computer will get out of phase with the other. Whereas XMODEM uses asingle statement monologue between each block, "C1" uses a multiple partdialogue. This makes "C1" about 3% slower than XMODEM, but this smalltrade-off in speed for accuracy will be well worth it the first time you runinto trouble with XMODEM.XMODEM commincations would look something like this:Xmit: Transmits BlockRec : "Good"Xmit: Transmits Next BlockRec : "Bad"Xmit: Transmits Same Block AgainIn "C1", the transmission would look something like this:Xmit: Transmits BlockRec : "Good"Xmit: Good block acknowledgedRec : Send next block for meXmit: Transmits Next BlockRec : "Bad"Xmit: Bad block acknowledgedRec : Send that block againXmit: Transmits Same Block AgainIn this type of transmission dialogue, neither computer can get out of sync,since should it receive the opposite response than it expects, it goes back togive the correct code word for the response it DID RECEIVE, thus regainingproper synchronization. Couple this with the "Statement and Listen Loops", andyou can readily see than communication would be hard to break down.Section C1-9------------Syntax Description------------------The following diagram should give you an understanding of the flow ofinformation between blocks:For a Good Block:Xmit: [Block]   "ACK"   [Next Block]Rec :       "GOO"   "S/B"For a Bad Block:Xmit: [Block]   "ACK"   [Same Block]Rec :       "BAD"   "S/B"Actually, the two are identical; the only difference is the substitution ofeither "GOO" or "BAD" as the response to the received block. Immediately afterreceiving the block, the receiving computer recalculats the checksum todetermine validity of the data. In the meantime, the transmitting computerstarts to wait for a "GOO" or "BAD" signal. Since it can "say" nothing untilit receives one of these codes, it merely waits. That may sound suspiciouslylike a good place to "hang up" the protocol, but the receiving end iseventually going to finish receiving the block, either because it timed outwaiting, or it finished collecting the correct number of bytes from thetransmitting computer.At that time, the receiving computer sends the appropriate code word ("GOO" or"BAD") and begins to wait for an acknowledgement ("ACK"). If it doesn'treceive the "ACK" in about one half second, it sends the "GOO" or "BAD" codeword once again. Meanwhile, the transmitting computer has been patientlyawaiting the reception of the "GOO" or "BAD" code. Once it receives it, ittransmits an "ACK" and starts to wait for an "send block" signal ("S/B"). Ifit doesn't get the "S/B" within about one half second, it sends "ACK" again.Back at the receiving computer, which is waiting for this "ACK" signal, itreceives it and sends the "S/B" signal and begins to wait for the block.Should it receive an "ACK" while waiting for the block, or receives nothing atall for approximately .5 seconds, it assumes that the transmitting computerhasn't heard the "S/B" and transmits it again. In the meantime, thetransmitting computer is waiting for the "S/B", and upon reception, startssending the block. The process has now started all over again.A quick analysis of this system will reveal that it's damned near impossibleto get any type of noise which could possibly mimick the code sequencesrequired. Also, no noise could stop the eventual completion of the abovesequence, since each computer is aways "sending and waiting". If two peoplekeep repeating their sentences over and over, and continue to

listen to theother person, even a noisy room couldn't stop them from hearing one anotherEVENTUALLY. Of course, some line noise is just so horrendous, that even thismethod of communication could fail. Then again, this type of noise would makeit damned near impossible for the user to be online in the first place, so itcan be considered an unlikely event. But, should one of the computers gooffline for any reason, we wouldn't want the other computer to keep loopingand looping until it died of old age.Although I haven't built in such protection into the terminal program Idistribute in the public domain, my BBS program does have abortion code.Should the protocol on the BBS have to go through the "Statement and ListenLoop" more than 24 times in row (which is hightly unlikely if the othercomputer is still online), it will abort the transfer. Similar code could beused in your implementation.Section C1-10-------------The End-Off Situation---------------------When the final block is transmitted, the high order part of the block numbershould be made HEX "FF" (255 decimal). This will inform the receiving computerthat this is the last block of data, and to expect no more. The question nowarises; how can both computers be 100% sure that the other is fully aware ofthe file completion? A fair question, but not one with a simple answer.When the transmitting computer receives the "GOO" for the last block, it canbe fairly certain that the receiving computer has received the final block,but it must inform the receiving computer that it knows this. It does so bysending an "ACK", but cannot be sure the receiving computer has received the"ACK" unless it gets the "S/B" signal back. Now, the transmitting computermust acknowledge the reception of the "S/B", but under the normalcommunications syntax, it would now have send a block. This is where the"End-Off" syntax comes into play; after receiving the "S/B", the transmittingcomputer sends back a "SYN" signal. In response to that receiving computersends it's own "S/B" signal, then waits for the final "S/B" from thetransmitting computer. Since it will not be responding to this code, it simplygoes into a wait cycle for approximately 5 seconds.If it does get the "S/B" within that 5 seconds, it ends immediately, butotherwise doesn't really care if it receives the code or not since at thisstage, there is a 100% assurance of both computers knowing things are Ok. Thetransmitting computer need only send three copies of the "S/B" code at thispoint, since, as stated above, there is full assurance that both computers arefinished. NOTE that the code words chosen for the End-Off situation are notnecessarily related to their appearant function.Section C1-11-------------Transfering File Type---------------------When transfering files from one computer to another it is often necessary toalso transfer the file type, but this must be known BEFORE the file is opened,and, therefore, before the protocol begins. "C1" does not impose any strictrules on what sort of information you transfer about the files, if any, butwhen writing a terminal program to communicate with one of my bulletin boards,the following should be done:Using a full implementation of the "C1" procotol (first dummy block, datablock, and End-Off), transmit a single byte of data corresponding to thefollowing file types:1 = Program File2 = SEQ File3 = WordPro FileTransmitting this single piece of data would require that TWO blocks be sent;the initial dummy block to set up the size of the first data block (of whichthere will be only one, size 8), and the data block itself, consisting of 7header bytes and the single file type byte. For other applications, one couldconceivable transfer much more information, including file name, file type,computer type, etc. It could even be possible to transfer multiple files,specifying the number and name of each file in this first transmission.Alternately, no one said you HAVE to use this first separate transmission; ifno information other the file needs to be transmitted, you just send the fileand nothing more.