@                         Using the Forth AssemblerWhat this file is:   A lot of the mail I have been getting about Blazin' Forth concerns theuse of CODE and ;CODE definitions, and combining assembly language withForth.  It recently occurred to me that a lot of people don't know how touse the Forth assembler - and are therefore missing out on a considerableamount of the power of the Forth language. So this is an attempt at atutorial in using Assembler in Forth, and in using it in the BFC inparticular.  Much of what I say here should be applicable to other 6502Forth implementations - but not necessarily all. Consider yourself warned.What this file is not:   This is *not* a course in machine language programming.  If you needinformation on the basics of using assembly language, then consult a goodtext - my personal favorites are the ones by Lance Leventhal, but there aremany good ones around.General Considerations:   Using CODE and ;CODE words are frowned upon in programs which attempt tobe portable.  In fact, the quickest way to insure that your programs are not83 Standard is to use the word CODE in them.   Having said this however, there is much to be said for incorporating afew code definitions in Forth programs.  Typically, a program will spendmost of its time in one or two words - by recoding these words in assembler,the overall speed of the program can be increased manyfold - 50% increasesor more are not unusual, and for very little work.   Also, the incompatibility is not as great as it would appear at first.It is probably safe to say that the most popular 8-bit personal computersuse the 6502 CPU (Apple II, C64 C128, Atari line etc.). As long as your CODEdefinitions are not accessing special hardware features (such as the SIDchip in the C64) most code definitions will work on all computers using thesame CPU. I have shared CODE definitions with friends who have Apples andAtaris running Forth, and I have never yet had to modify one.   Finally, Forth is the easiest language there is to combine with machinelanguage.  Many of the hassles and problems ordinarily associated withcombining high level languages and assembler simply do not occur in Forth.In other languages, the typical process is this:   1: Write the high level program, and debug.   2: Figure out where you need to speed things up.   3: Fire up an editor, and write some assembly code.   4: Assemble the file and - usually - run a loader program on that output.   5: Load the Editor, and modify your higher level source.   6: Compile your higher level source.   7: Link the compiled program and the loaded assembly language program.   8: When it doesn't work (which it won't, at first). Go back to step 3.   There are also inevitable problems in passing parameters to and from thehigher level code, and the inevitable problem of where to put the machinelanguage. If you have ever done much of this sort of thing, you know what aheadache these problems can be.   These last two problems are usually the most difficult, and, you will besurprised, and possibly relieved to hear that they don't occur in Forth. Atall. Ever.   Combine that with a resident assembler and a resident editor and aresident compiler, writing Assembly language in Forth becomes almostridiculously easy.A Quick Example:   Since I have taken up so much of your time with the above advertisement,you would probably like me to put my money where my mouth is. Here is aquick example of what I was talking about above.  I hope it's not tootrivial, but the idea I want to get across here is the ease of combiningAssembler with Forth. First, a high level Forth Program:: SHIFTLEFT ( -- N2 N1 ) // shift N2 left N1 times      0 ?DO  2* LOOP ;: SHIFT-4 ( Just show what a left shift is )      100 0 DO  I 4 SHIFTLEFT .  LOOP ;Multiple left shifts are fairly common, and so SHIFTLEFT is likely to be ahandy word in certain applications.  As it is, it will run pretty quickly.But perhaps, for certain speed demons, not quickly enough, so you decide torecode the SHIFTLEFT primitive in Assembler:CODE SHIFTLEFT ( -- N2 N1 )  // shift N2 left N1 times   BOT LDA, TAY, BEGIN, 0 # CPY, 0= NOT  WHILE,      SEC ASL, SEC 1+ ROL, DEY, REPEAT,   POP JMP, END-CODEIf you are used to conventional assemblers, this probably looks prettyweird.  The important thing to notice here is that *only* SHIFTLEFT haschanged - SHIFT-4 (or any other word which uses SHIFTLEFT) will work just asit did before, with the only change being the overall increase of speedwhich machine language naturally brings to any situation.  Notice also thatwe didn't have to worry about where to put the code - it goes in the samespot our higher level SHIFTLEFT went. You will also discover, if you typethis example in, that you don't have to call the assembler. This is alltaken care of for you.  As far as you, another user, or other procedureswhich use SHIFTLEFT are concerned, there is no difference between using thehi-level SHIFTLEFT and the CODE SHIFTLEFT.The Structure of a CODE definition:   It's pretty straightforward. They all look the same:CODE [name] [assembler mnemonics] END-CODENote the similarity to a colon definition:: [name]    [forth words]         ;How to Exit a Code Definition:   One thing you must remember is that you have to explicitly leave a CODEdefinition by doing a JMP, to another code level routine.  This is probablythe single most common error made by newcomers.  Possibly it is caused bymaking a false analogy between the higher level ; and the code levelEND-CODE. While the Forth word ; does in fact get you back to where you camefrom, END-CODE does not. In fact, END-CODE does nothing at all at run-time.You can exit a CODE definition by doing a JMP, to any of the following:NEXT POP POPTWO PUSH or PUT .  These are described below:NEXT   NEXT is commonly called the address interpreter. It is the word that isresponsible for the execution of all Forth words.  ALL words in Forthultimately end up here.  Doing a NEXT JMP, will cause the current codedefinition to stop and return to the word that called it.  All of thefollowing exit points end with a jump to NEXT .  In what follows, rememberthat a "stack element" refers to a 16 bit quantity -- i.e. two bytes.POP   POP first drops the first element of the stack, and then jumps to NEXT.Same as DROP in hi-level forth.POPTWO   POPTWO drops the top two elements from the stack, and then jumps to NEXT.Same as 2DROP in hi-level.PUSH   PUSH lets you leave a result on the top of the stack.  PUSH expects thelow byte of the new top of stack to be on the return stack, and the highin the accumulator. PUSH will leave these as the new top of the parameterstack (the former top will then be the second element). PUSH then callsNEXT. Since this routine is somewhat more complicated than the others hereis a typical sequence:      PHA, ( push low byte to return stack )      TYA, ( assume high byte is saved in Y register, move it to the A reg)      PUSH JMP, END-CODE ( parameters set, so jump to PUSH)PUT   PUT replaces the top of the stack with a new value.  You setup PUT inthe same way as you setup for PUSH - the new lobyte is pushed to the returnstack, and the new hibyte is in the accumulator before the call to PUT. Theonly difference between the two is that PUT replaces the present top of thestack, while PUSH creates a new top of stack. Register Usage:   Since Forth uses two stacks, and the 6502 CPU only implements onehardware stack, the parameter stack must be "artificially" maintained. Inthis implementation (as in most) it is located in the zero page, and the Xregister is used as the parameter stack pointer. The machine stack isForth's return stack. Therefore instructions which affect the X register orthe machine stack should be used with care.  At entry to a CODE defintion,the X register points to the top byte of the parameter stack.  This stackgrows downward in memory, so decrementing the X register will make roomfor another element on the stack, and incrementing the stack pointer willremove an element from the stack. Here is a diagram which shows thesituation when two elements are on the stack:         Hi Memory      **************      *   hibyte2  *      *   lobyte2  *      **************      *   hibyte1  *X --> *   lobyte1  * Top of Stack      **************Notice that the two bytes which make up one stack entry are stored in theusual 6502 order, with the lobyte lower in memory. To remove the top elementof the stack, we can define the code word DROP:CODE DROP ( N -- )  INX, INX, NEXT JMP, END-CODEWhich in fact, is exactly the the way DROP is defined in the BFC. After DROPhas been executed, the stack looks like this:        Hi Memory      *************      *  hibyte2  *X --> *  lobyte2  * New top of Stack      *************We will return to this topic later, when we talk about accessing theparameter stack in more detail.  For now, the main point is to remember thatwhen Forth starts executing your code definition, the X register willcontain a pointer to the top of the stack. You can use the X register toaccess the stack, or to remove elements from the stack, but when your codedefinition is finished, other Forth words, and the Forth system itself isgoing to expect the X register to contain a valid stack pointer, so don'tchange it wantonly. You should also remember that since each stack entry istwo bytes, only even multiples of the INX, or DEX, instruction make sense.(I.E. INX, INX, INX, INX, not INX, INX, INX, - the first will drop two stackelements, while the second will drop 1 and 1/2 stack elements - and causethe Forth system to behave oddly.)Both the Accumulator (A reg) and the Y register may be freely used. The Aregister will contain

garbage, and must be initialized, but the Y registeris guaranteed to be 0 on entry to your code, and you may take advantage ofthis fact or not, as you wish.  Here is a short code definition that willleave the value of -1 (Forth's canonical TRUE flag) on the stack. It usesthe PUSH routine described earlier.CODE TRUE ( -- -1 )   DEY, ( Y REG now holds $ff)                      TYA, PHA, PUSH JMP, END-CODEHere is an even shorter definition which will leave a 0 on the parameterstack:CODE FALSE ( -- 0 )  TYA, ( set A register to 0 )                     PHA, ( set up for PUSH )                     PUSH JMP, END-CODELife being what it is, you will often wish you could use the X register.There is a way.  You can use the system storage location XSAVE, totemporarily save the value of the X register while you are doing otherthings. You must remember to restore the X register before exiting, however.A typical sequence is:  XSAVE STX, ( stuff that changes x ) XSAVE LDX, NEXT JMP, END-CODEWhy it looks so strange....   The main reason the Forth Assembler looks so strange is that it isreverse polish, like all of Forth. Operands *preceed* the operators. Hereare some examples that should make it clear:Conventional Assembler           Forth's Assembler=====================            ================   LDA # 0                          0 # LDA,   ROL A                            .A  ROL,   STA ADDRESS,X                    ADDRESS ,X STA,   STA (ADDRESS,X)                  ADDRESS X) STA,   LDA (ADDRESS),Y                  ADDRESS )Y LDA,   JMP (INDIRECT)                   INDIRECT ) JMP,   JMP ADDRESS                      ADDRESS  JMP,   LDA ADDRESS                      ADDRESS  LDA,While admittedly unusual, it does make the best use of the stack at assemblytime.The other major difference is that the Forth Assembler does not use labels.There are no branch instructions - the Forth Assembler uses a structuredcode approach to control flow. It does this by using analogues of thehi-level IF THEN ELSE etc. to control the flow of your CODE definition. Totake advantage of this, you must specify the condition code you want tested.You specify this condition code by using any of the following words:   CS  test if carry set   0<  test if negative flag set   0=  test if zero flag set   VS  test if overflow flag set   You can follow these condition code specifiers with not, to test for theopposite condition:   CS NOT test if carry clear   0< NOT test if negative clear   0= NOT test if zero flag clear   VS NOT test if overflow flag clearBelow is an example of a possible definition of 0= , which leaves true ifthe top of the stack is 0, and false (0) if it is anything else:CODE 0= ( N -- FLAG ) BOT LDA, BOT 1+ ORA, 0= IF, 255 # LDA, ELSE, 0 # LDA,THEN, PHA, PUT JMP, END-CODEIn the above code, we first test for 0 by ORing the two bytes which make upthe top of the stack together. The result will be zero only if both arezero. We then test the zero flag (with 0=). If the byte is 0, we LDA with255, otherwise, we LDA with 0, and replace the top of the stack with theflag by jumping to the PUT exit routine. Note that we could make thisdefinition much shorter by taking advantage of the fact that the Y registeris zero at entry:CODE 0= ( N -- FLAG ) BOT LDA, BOT 1+ ORA, 0= IF, DEY, THEN, TYA, PHA, PUTJMP, END-CODEYou can use the same type of tests to do conditional loops. Here is a donothing example that simply wastes some time in a loop:CODE WAIT ( -- ) BEGIN, DEY, 0= UNTIL, NEXT JMP, END-CODEThis simply decrements the Y register until it becomes zero.  In theoriginal Forth assembler for 6502 machines, the BEGIN, UNTIL, structure wasthe only one available. The BFC has extended this to include BEGIN, WHILE,REPEAT, and BEGIN, AGAIN, . The BEGIN, AGAIN, loop is infinite - you mustJUMP out of it in the middle somewhere.Accessing the Stacks.   Typically, most routines only need to access the top two elements of theparameter stack. Since this is so common, special words have been providedto make life easier here. BOT references the top of the stack (which islower in memory, and so the BOTtom of the stack). SEC references the SECondelement of the stack. It's important to remember that a Forth stack entry is16 bits, or two bytes, so to obtain the whole stack element, you need to dotwo fetches or stores. Here is a sample implementation of DUP:CODE DUP   ( N -- N N ) BOT LDA, PHA, BOT 1+ LDA, PUSH JMP, END-CODEFirst we fetch the low byte of the top of the stack with BOT LDA, . This ispushed onto the return stack, as required by the exit routine PUSH . Next weget the high byte with BOT 1+ LDA, . That's all there is to it. As anotherillustration, here is an implementation of OVER:CODE OVER ( A B -- A B A ) SEC LDA, PHA, SEC 1+ LDA, PUSH JMP, END-CODEThe actual addressing mode being used here is "zero-page x". Inconventional assembler, BOT LDA, would be written LDA 0,x while BOT 1+ LDA,would be written LDA 1,X . To access deeper stack elements, you can keepadding values to BOT or SEC , or you can use the addressing mode explicitly:BOT 4 + LDAor4 ,X LDAWhile not used often, it is also possible to address directly into thereturn stack. Typically, you would access the return stack using the PLA,instruction. This has the side effect of altering the stack pointer, and youcan also only access the top of the return stack.  To access arbitrarybytes, you can use RP) . To do this, you must first save the X-Register inXSAVE and then execute TSX, which will move the stack pointer into theX-Register. You can then do RP) LDA, which will fetch the current top of thereturn stack. To get deeper into the return stack, offset RP). Below is anexample which non-destructively moves the address on the return stack to thetop of the parameter stack:CODE GET-RETURN-ADDRESS  XSAVE STX,  TSX, RP) LDA, PHA, RP) 1+ LDA, XSAVELDX, PUSH JMP, END-CODEOne of the easiest and quickest ways to crash any system is to garbage thereturn stack. If you need to access the stack, go for it, but use care.SETUP and N   It is often useful to be able to access absolute memory locations.  Tothis end, Forth provides an 8 byte temporary data area which is referred toas the N area. You may initialize this area yourself, or you may call SETUPto move stack elements to the N area. To use SETUP, you must load theaccumulator with the number of stack elements you want to move (NOTE: thenumber of stack elements, NOT the number of bytes) and then do a JSR toSETUP. SETUP will pop the elements off of the stack, and move them to the Narea.  Since there are only 8 bytes in the N area, you have room for at most4 stack elements. As a simple example, the following will pop the top twoelements off of the stack, and move them to the N area:CODE MOVE2  2 # LDA, SETUP JSR, END-CODEThe previous top of stack (BOT and BOT 1+) will be stored at N and N 1+ ,while the second element (SEC and SEC 1+) will be at N 2+ and N 3 +. Oncethey have been moved there, you can carry out operations on them, or usethem in the indexed indirect Y addressing mode. ( LDA (N), Y in conventionalassembler, or N )Y LDA, in Forth assembler.)  By far the most common errorin using SETUP is to forget that it also pops the elements of the stack asit moves them.;CODEIf CODE is the Assemblers equivalent to the hi-level colon, ;CODE is theassemblers equivalent to DOES>. As an example of the use of ;CODE, we willwrite our own versions of CONSTANT - one in high level forth, using DOES>,and one in low level, using ;CODE. First the hi-level definition:: CONSTANT  CREATE , DOES> @ ;As a quick review, remember that CREATE (or a word that uses CREATE) createsa dictionary entry for the next word in the input stream. Words created inthis way all have the same run-time behaviour - they leave the address oftheir parameter field on the stack. Note that CREATE by itself does notallocate any parameter field space in the dictionary, you must do thisyourself by using C, , or ALLOT . The DOES> word allows you to manipulatethe values in the parameter field, essentially allowing you to define aspecial set of related words which share the same run-time behaviour. Letswalk through the process:10 CONSTANT TENWhen Forth executes the above line, the number 10 will be left on the stack,and CONSTANT will be executed. The first word in CONSTANT is CREATE, whichwill take the first word it finds in the input stream (TEN in this case) andcreate a header for it in the dictionary, with its associated link field,name field and code field. The code field written by CREATE will cause theaddress of the first byte of the parameter field to be left on the stackwhen TEN (the word defined by CONSTANT) is executed. Next , is executed,which takes the top of the stack, and compiles this number in thedictionary. DOES> does something mysterious (more on this later) whichcauses the words following DOES> to be executed at run-time. This is thecompile time behaviour of CONSTANT. Now, when TEN is executed, the run-timebehaviour of CONSTANT will occur. The parameter fields address is pushedonto the stack, and then the words after DOES> are executed. In this case,the single word @ will be executed, which replaces the address on the top ofthe stack with the value at that address.TEN . 10 OKNow for the ;CODE version:: CONSTANT  CREATE , ;CODE  2 # LDY, W )Y LDA, PHA, INY, W )Y LDA, PUSH JMP,END-CODENotice, by the way, that no concluding semi-colon is required when you use;CODE.When ;CODE is executed (for example, by typing 10 CONSTANT TEN), it willREWRITE THE CODE FIELD OF THE WORD BEING DEFINED! This is extremelyimportant to remember.  All Forth words have code fields that define theirrun time behaviour, with colon definitions

all sharing the same code field,variables sharing the same code field (different, of course, from that ofcolon definitions and constants) and so on. When CREATE executes in ourCONSTANT definition, it creates a code field that contains the address of aroutine which will push the address of the word being defined to the stack.When ;CODE executes in our CONSTANT definition, it will search out this codefield, and re-write it with the address of the machine language routinewhich immediately follows ;CODE. This means that the only run time behaviourof a word which contains a ;CODE termination is determined by you. Eventhough there is a CREATE in the lo-level definition of CONSTANT, it will notdeposit the address of the parameter field on the stack, because ;CODE haschanged the code field to point to our machine language routine. Anillustration may make this clearer:Header created by CREATE:*************** Link Field **************** TEN        *  Name field - stores ASCII characters of defined name.***************  $0900     *  $0900 = address of routine to push parameter field on stack*************** 10         *  Parameter field - stores value of constant**************Note that $0900 is just an example address - don't try jumping to it in yourown code.When a word defined by CREATE is executed, the machine language routine at$0900 (the address in the code field of the word) will be executed. Thisroutine is what causes the run-time behaviour of CREATE defined words - itpushes the address of the parameter field to the stack. The aboveillustration shows the state of the dictionary after the CREATE and , partof CONSTANT have been executed. Now, suppose that the address of the firstinstruction following ;CODE in CONSTANT ( the 2 # LDY, ) is at $8000 . When;CODE executes in the course of defining a new constant, the header willlook like this:Header after ;CODE has been executed.*************** Link Field **************** TEN        *  Name field - stores ASCII characters of defined name.*************** $8000      *  $8000 = address of your machine language routine*************** 10         *  Parameter field - stores value of constant**************So, as always, when Forth executes TEN, the first thing to be executed isthe address of the routine in the code field, which no longer points to theCREATE run time code, but to your machine language code.Hope that is all clear enough. Now, to the particulars of the ;CODE part ofour low level CONSTANT definition (note that the following is implementationdependent. Most present day Forths use an implementation along these lines,but not all.)Forth depends on two registers for its performance, the IP and the Wregister. When Forth is implemented on processors with more registers,typically a processor register will be used for these Forth systemregisters. However, there are not enough on the 6502, so zero page locationshave been used instead.  W will contain the address of the code field of theword currently being executed. For example, if the code field of TEN islocated at $9000, the W register will contain $9000, or the address of theaddress of the routine to be executed. The ;CODE part of CONSTANT uses thisfact to access the parameter field of the current word. Since the code fieldof an address, is always two bytes long, the first byte of the parameterfield is located at $9002. Given this information, it becomes a simplematter of indirectly indexing from W. Loading the Y register with 2, andthen performing W )Y LDA, (LDA (W),Y in conventional assembler) will get thefirst byte of the constants value. The next byte is at $9003, so we simplyincrement Y, and indirectly index again to get the next byte. The rest ofthe definition consists of setting up for PUSH, which was covered earlier.Phew! Hope that was clear enough. I think you can see that indexing from Wcan get you any byte in a definitions parameter field. W 1- contains anindirect JMP instruction, so you can also vector control to other routinesby storing the address of the address of the routine in W, and then doing aW 1- JMP, . This technique is rarely used, however.There is an intimate relationship between the IP register and the Wregister, and an effective use of these registers depends on a clearunderstanding of how they work in the Forth system as a whole.  Both of themare changed by NEXT. When NEXT executes, the IP will point to an addresswhich contains the code field address of the next word to be executed byForth. NEXT fetches this address, and stores it in W. It then bumps the IP(which stands for Interpretive Pointer) by two, so the next time around itwill be pointing to the next word to be executed. Finally, NEXT does a JMPto W 1-, which causes an indirect jump to the address pointed to by W.The IP is most useful for accessing things like inline data structures,(character strings, and the like.) Since the IP is incremented by NEXT, thecode fragment:    IP )Y LDA, PHA, INY, IP )Y LDA, PUSH JMP,Will push the address which contains the address of the next Forth word tobe executed to the parameter stack. (This will be the third and fourth bytespast the address of the current word being interpreted.) Obviously, bystoring new values in the IP, and then jumping to NEXT , you can also forcethe execution of a particular definition.  This technique is also rarelyused. Typical uses for the IP are mainly accessing inline data.Accessing Variables from Code definitions.   Since variables leave the address of their parameter fields on the stack,it is a simple matter to access these variables from CODE level definitions.Here is an example:VARIABLE FOOCODE FOO+1   FOO LDA, CLC, 1 # ADC, FOO STA, CS IF, FOO 1+ INC, THENEND-CODEThis is a common way to increment the value at a memory location.  Theassembler also provides a way to access USER variables, using the UP (userpointer constant). User variables must be accessed using an offset, so the)Y addressing mode  is recommended. You will need to know the offset of theuser variable you need to access in advance, of course.MISC.   Since the Assembler is co-resident with the FORTH system, all the power ofFORTH is available to you when using the Assembler. As one example:   FOO @ 2 3 */ # LDA,will initialize the accumulator to two thirds of the value stored in thevariable FOO. (Of course, this must be 255 or less.) Occasional conflictsmay arise, however. In particular, a common error is to confuse theassemblers 0= and 0< words with Forths - they are not the same. If you wishto use the Forth versions while assembling, you must explicitly enter theFORTH vocabulary, do your Forth thing, and then re-enter the assemblervocabulary. Such conflicts are rare, and usually easily recognized.The Forth assembler uses the standard MOS mnemonics for the 6502 op-codes,but each mnemonic has a ',' attached. Thus, in forth, we write LDA, BRK, orJMP, not LDA BRK and JMP. Also, the Assembler conditionals use the sameconvention - IF, ELSE, THEN, and not IF ELSE THEN . A common error is toomit the comma from one or more of these conditionals.  I am personally notwild about this convention, but the first Forth assemblers used them, andnow we are stuck with it.You may have noticed that I have not used the JSR, RTS, instructions. Thisis because typically, CODE definitions are called from higher level words,and must end with a JMP, to NEXT or the equivalent routine. It is possibleto write programs which are structured in this way. Typically, thesubroutines are given names with CREATE, and then called from a higher levelcode definition. Here is an example:CREATE BLETCH  ASSEMBLER  NOP, NOP, RTS,Note that when we do this, we must invoke the Assembler vocabularyexplicitly. BLETCH would be typically called from a higher level CODEdefinition:CODE THE-GREAT-RTS-HACK  BLETCH JSR, NEXT JMP, END-CODEThis is only occasionally necessary however - usually when writing code thatis extremely time critical, such as graphics code.Finally, it is possible to exit a code definition by jumping to yet anothercode routine. You must remember to restore the X register (if you havealtered it) and it is also wise to reset the Y register to 0, since manycode level definitions will assume that this is the case. As a simpleexample, here is a word that simply simply does a JMP, to Forth's KEYroutine:CODE CALL-KEY  ' KEY @ JMP, END-CODEThe tick (') gets the address of KEY's code field, and the @ fetches theaddress of the routine which is stored in KEY's code field. This is therecommended technique. It is more portable, and also safer than others whichone sees.  Note that you cannot call hi-level Forth words using thistechnique - only code level definitions may be called in this way. If youthink you want to exit a CODE definition with a call to a higher level Forthword, think again. If you still need to do it, then start tinkering with theIP and W registers.I think that about covers it. I have tried to cover all of the basics, andmany of the more advanced techniques in combining assembly language andForth. If some of it seems obscure, it is probably my explanation, sincehacking in CODE is really no more difficult than hacking in any language -it just runs faster.Good LuckAndHappy Hacking!SDB