The program, nuterm.bin, is a bare bones terminal program, written in BASIC,to illustrate the technique of saving an ASCII text file to diskette withoutsending holdoffs to the host (CONTROL S, e.g.).  Because no error checkingprotocol is used, the method is notrecommended for program files.  But itproduces minimum online time.  If contiguous characters are received, they redisplayed and saved to a sequential file in essentially real time.  No buffer,other than the 255 byte, RS-232 receiver buffer, is used.  Thus, since theprogram ma be easily modified, the addition of a capture buffer will permitboth saving to diskette and buffer capturing for later printing.  It has beentested with a C-64C, a 1670 modem, and a 1541-II drive at 300 and 1200 baud.When LOADed and RUN, it transfers 515 bytes of ML to the region from $C000 to$C202.  The BASIC program itself occupies $0801 to $0D6C.  Any line numbergreater than 20 and less than 2020 may be deleted, edited, or added.  If theprogram is reSAVEd, be sure to make line 10 end in GOTO 2020 rather than GOTO0020.  Since the program was designed for use with CIS, F2$, F4$, and 6$ arestring variables which should be redefined as 'phone number, user ID, andpassword (unless your password happens to be number/modem).  After answeringprompts for FILENAME of file to be SAVEd and baudrate, pres F1 to enterterminal mode.  You are then in an ML loop.  You would usually press F2 to dialhe CIS 'phone number, press the STOP key for a control C when connection ismade, and follow with F4 for user ID and F6 for password.  Press F3 to returnto menu mode.  If a text file is to be SAVEd to diskette, request to REAd thefile.  After entering the CIS FILENAME,don't press return.  Press F5 instead.You should see the text on the screen and be SAVing to diskette.  The sporadicbreaks in printing to the screen, during drive read/write head activity, do notrepresent hold offs sent to the host.  As soon as the drive sets the programfree, the characters that had built up in the receiver buffer are rapidly readout.  At the end of the file, first press F7, then RETURN.  ONLY THE F7 KEY ISOPERATIVE DURING A SAVE.  PETASCII/ASCII and ASCII/PETASCII conversion is used.Again, this is just a demonstration of a technique whih may not be in commonuse.  The program hardly quallifies as a finished product.  It was designed forease of modification.  All suggestions,criticisms and comments are welcome.                    John Iannetta                    73510,2152