Notes on portprn.imgPortprn.img is a machine language routine.  Load it ,8,1 and SYS 4096 ($1000)The STOP key will break it at any time.The idea behind this program is a fairly simple one:  The C64 accepts input,one line at a time, over the user port to which is connected an RS 232interface.  It translates the input from pure ascii into petscii as the datais received, and then prints it to a printer (device 4) on the bus.  I wentto this effort because I bought a second computer, which has a serialinterface, and did not want to spend money on another cheap dot-matrixprinter.  The 1526 is a perfectly sound, if not lightning fast, textprinter.  The setup serves my purposes, kludgy or not.  Although I have hadproblems getting it to work with a screen dump, it works fine when printingunder program control.Screen notes:The screen shows the status of both the (serial printer) bus and the (RS 232user) port.  The checkmarks indicate the state (open/closed) of each.  Inaddition, the error codes are read and sent to the screen as characters.Error checking per se is not done, rather, the results in the Accumulator orin the status byte are sent -- good, bad, or indifferent -- straight to thescreen.  It's a lazy way out, but if things aren't working, at least thecodes are there to be analyzed.  They are in the following order:RS 232 Status   open:  Error result in Accumulator and readst byte from kernal OPEN   init:  Error result in Accumulator from kernal CHKIN   close: blank; never closed.Printer Status   open:  Error result in Accumulator from kernal OPEN   init:  Error result in Accumulator and readst byte from kernal CHKOUT   close: Readst byte from kernal UNLSN, error result in Accumulator  and          readst byte from kernal CLOSEOf course, what you see are the screen codes (PRG Appendix B).Programming notes:The RS 232 interface is set at 1200 bps, even parity, seven data bits, onestop bit.  The x-line handshake is implemented, although the port buffer isalmost always empty when operating at ML speeds.Alphanumeric and punctuation characters are translated; carriage return andform feeds are fed straight to the printer.  All others, including tabs andlinefeeds, are stripped.This is only significant ML program I have ever written.  I prefercompilers, but because Oxford Pascal does not recognize the user port atall, this seemed a natural application.  The source code is commented andmakes liberal use of symbols as I made notes to myself along the way.  Itwas originally supposed to be a spooler, but that seems to be impracticalfor reasons outlined below.  My choice of location was purely arbitrary andprobably less than ideal.  It wouldn't be very difficult to change, were youso inclined, by re-assembling with a different origin.  I used the EAassembler by Lew Lasher available (or was) on this SIG.  The buffered lineis stored at $7000, and probably should be moved closer to the code, Iguess.I don't pretend that it is the most elegant code ever written, but I thinkit may serve as a starting point if you are thinking of writing acommunications program, or have a situation similar to mine.  I have readthree books on ML programming, including both of Richard Mansfield's highlyacclaimed volumes.  Helpful as they were, communications I/O was not in anyof them.  Nobody discusses the difference between CHROUT and CIOUT, forinstance, and for me there was a lot of trial and error.  For this reason,it is my hope that the source code will be instructive.Why not a spooler?  Both the bus and the port communicate bit-by-bit (no punintended).  Assembly/disassembly of the BYTES used within the C64 to theBITS it needs to communicate is done in zero page by the CPU.  Our friendthe 6510 does one thing at a time, does it very well, and then goes on (likeMajor Winchester).  Ergo, it cannot address both the port and the bus at thesame time, as they need the same ZP routines.  This limitation made itnecessary (as I understand it) to close the bus down completely whenprinting.Hardware note:I constructed a null modem cable crossing pins 2/3, 4/5, 6/20. Crossing 5/20should work as well.  The software drops DTR (20) when it receives acarriage return, causing the sending machine to hold off.