INSTRUCTIONS FOR USING:        VKYDOW.SEQ        VKYTOK.SEQ      AND OTHER VKY???.* FILES   20   REM THESE TWO PROGRAMS WILL TAKE A BASIC PROGRAM LISTING   30   REM (IE ONE THAT LOOKS LIKE THIS PROGRAM) AND WILL COPY   40   REM IT ONTO THE DATASETTE RECORDER AS DATA. THE TOKEN PROGRAM   50   REM WILL TAKE THE DATA FROM THE VIC20 DATASETTE RECORDER AND   60   REM CONVERT THE DATA TO A PROGRAM (INSIDE THE VIC20).   70   NREM FOR THE DWNLOD PROGRAM::: THE FUNCTION KEYS PERFORM AS   80   REM FOLLOWS- F1:CONTROL C     F3:CONTROL P     F5 CONTROL Q   90   REM F7:CONTROL S       F6:START SAVING EVERYTHING ONTO THE  100   REM DATASETTE RECORDER.    F8:STOP SAVING ON DATASETTE.  110   REM THE TOKEN PROGRAM WORKS AS FOLLOWS---  120   REM IT WILL LOOK FOR THE DATA THAT WAS SAVED ON THE  130   REM DATASETTE RECORDER.IT WILL THEN PRINT EACH LINE ON THE  140   REM SCREEN. AFTER PRINTING EACH LINE,  150   REM IT WILL ASK YOU IF YOU WANT TO SAVE  160   REM IT OR SKIP OVER IT.THAT WAY ANY GARBELED LINES YOU  170   REM CAN SKIP AND ENTER THEM YOURSELF LATER  180   REM BASICALLY, ANY LINE IS ENTERED,AND GARBAGE(LIKE MISC  190   REM CARRIAGE RETURNS,PHONELINE GARBAGE,ETC.)CAN BE ELIMINATED  200   AFTER TOKEN HAS LOADED ALL OF PROGRAM/DATA FROM THE RECORDER  210   REM IT WILL THEN ERASE ITSELF,LEAVING ONLY THE CONVERTED  220   REM PROGRAM IN THE VIC20:(IE-MAKE SURE YOU MADE COPIES BEFORE  230   MEM YOU RUN IT SINCE IT WILL ERASE ITSELF  240   REM A NOTE TO THOSE WHO USE THIS PROGRAM. IF YOU LIKE IT,  250   REM OR EVEN IF YOU DON'T, I WOULD LIKE TO HEAR YOUR COMMENTS  260   REM THESE PROGRAMS WERE WRITTEN AND TESTED ON AN UNEXPANDED  270   REM VIC20.THE ONLY TIME I USED ANY EXPANSION WAS FOR THE  280   REM UPLOADING PROGRAM I WROTE TO UPLOAD THESE 3 PROGRAMS*************************************** this is a list of many different items. you should find most of them useful. 1. for better operation of dwnlod.vky :    a. change line 330 to       'a=f%(asc(a$)):printk$;chr$(a);:ifa=ethenpokey,d'    b. change line 340 to       'ifmthenprint#1,chr$(a);:ifpeek(t)=>lthen500'    c. change line 430 to       'i=peek(t):print#1,chr$(31);:ifpeek(t)>ithen430'    d. basically,you are  changing from < a=asc(a$) >       to  a=f%(asc(a$)) >,changing from       < chr$(f%(a)) > to < chr$(a) > and        changing from < chr$(10) > to < chr$(31) > 2.there are basically 2 types of programs   in the database.sequential and program. if   it has 'img' after the program name, then   it's a program file. when you use the 'read'    command, if what you see looks like a program,    (ie,looks like what you see when you type 'list' on your computer),    then it is a sequential file,and can be downloaded by my program.if what     you see looks like gibberish,then you cannot    download it with my program (ie,it has 'img'   after the program name).my program will only   download sequential files (ie they look like a regular program   listing).to download the other types,(ie   program files,having 'img' extension),you   need a program that uses compuserves protocols. 3. method of downloading a file,program listing,   data,messages,etc. with my 'dwnlod.vky program    a. use 'read' command and see if it looks       like a regular program listing. if it doesn't,       my program won't download it.    b. use the 'READ' command again, but do not-       repeat, donnot push the return key.    c. while hlding the shift key,push the f5       function key(it's actually the f6 key since       you pushed the shift).now you have activated       the downloading function.    d. now push the 'return' key so that the 'typ'       command will be executed.    e. you will notice that while the 'typ' is being executed,       (ie the program listing is being listed on       the screen),it keeps stopping every 7 to 15       seconds.that's because the vic is saving it       on the datasette recorder.    f. when the screen is done listing,you will       recieve the 'sig/acess:' prompt. at this time,       push f8 (hold the shift and push f7 function key).       this will deactivate the download function.    g. if there is another program,file,message,       or whatever that you want to download,just       start againg at instruction a (using the      'read' command).    h. you can save continously as much as you       want up to the end of the casette tape.       (and even more if you turn the tape over). 4. what you see on the screen is exactly what will be    downloaded by my program.you can download    messages from bulletin boards, etc.    since you download what you see,special    control cods [ie print quote clear/home endquote]    won't work.this is why my programs are    written using the 'chr$(xx)' commands to     replace the special vic20 control codes    (like the 'clear screen','reverse video','change     character color',etc. 6. after having been told by c64 owners that my programs    work on their machines,i bought the c64     programmers reference manual.since all the    memory locations (that i used)are the same    in the c64 as they are in the vic20,i have   change my original opinion (that they    wouldn't work in the c64).looks like they   do work ok in both machines.if anybody has   any opinions,comments,or additional info,   it will be appreciated. 7. if you downloaded messages,there is a program    to allow you to just read back onto the     screen what you downloaded. the program name    is 'seedld.vky'.it should be on the     database within a day or two of this. 8. if you have a lot of lines,screen interference,etc.    on your screen from the vic20,here's an easy    method i used to eliminate about 95% of it.    take the video cord that comes out of the    vic20(and goes to the modulator box),   and wrap it around your hand about 4 or 5 times   (ie a circle of about 3 inch diameter of    about 5 turns).i then taped this together   to hold the loops together.since this shortened   the cord to the tv,i used an eight foot audio   cable to connect to the tv.if the audio   cable lay in front of the tv,i still got some   interference,so i moved the cable to the   side of the tv.now it looksvery good. 9. there is another program for uploading    that i wrote.basically,it downloads the     same as my downloading program,but it will    also upload.that one needs additional     memory to work though (i'm working on    an upload program for the enexpanded vic20 now).    the uplod program uploads a line of text in about    4 seconds. i'm using the upload program    to enter all this into the vic20 (with 16k expander)    before i sign onto compuserve.    there are a couple of minor bugs in it which i'll fix in   another 'errata' file later).   the uplod program has a couple of lines that   need to be changed if your expansion is other   then 16k,but that should be covered in the   instructions that come with the upload   program.again, any comments,questions,opinions,   etc. are appreciated.(although it may take   me several days to get back to you)   terry [73465,1635]***************************************THIS IS A LIST OF THE MOST COMMON PROBLEMS AND QUESTIONS FROMUSERS.Q1. I CAN'T GET THE PROGRAM TO WORK. WHAT SHOULD I DO?A1. I DON'T KNOW BECAUSE I DON'T KNOW WHAT YOUR PROBLEM IS.FOR ME TO MAKE AN EDUCATED GUESS AS TO WHAT IS CAUSING YOURPROBLEM AND WHAT THINGS TO CHECK, I WOULD NEED MORE INFORMATION.DOES YOUR SCREEN SAY 'READY'? DOES THE LIGHT ON YOUR MODEMLIGHT UP (TO INDICATE THAT YOU HAVE DETECTED A COMPUTER) ATTHE PHONE NUMBER THAT YOU DIALED? ARE YOU ABLE TO GET ANYPRINTING ON YOUR SCREEN? DO YOU ONLY HAVE A PROBLEM WHEN YOUARE TRYING TO DOWNLOAD? DO YOU HAVE ANYTHING OTHER THEN ADATASETTE,1541 DISK DRIVE,OR 1525 PRINTER HOOKED UP TO YOURCOMPUTER? IF YOU ARE NOT USING A VIC20, WHAT KIND OF COMPUTERIS IT? DO YOU RECIEVE ANY PRINTED ERROR MESSAGES ON THE SCREEN?(EXAMPLE-SYNTAX ERROR,SUBSCRIPT OUT OF RANGE,DEVICE NOT PRESENT,TYPE MISMATCH,ETC.). IF YOU RECIEVED AN ERROR MESSAGE,WHAT LINENUMBER DID IT PRINT THAT THE ERROR WAS ON (THAT'S VERY IMPORTANT).IS THE PROBLEM LATER ON WHEN YOU TRY USING ONE OF THE OTHERPROGRAMS (LIKE TOKEN.VKY ETC.)? *** IF YOU DON'T GIVE ME ENOUGH INFORMATION I CAN'T REALLYHAVE ANY IDEA WHAT YOUR PROBLEM ACTUALLY IS RELEVANTTHAT YOU CAN THINK OF.(YOU CAN'T TELL ME TO MUCH)Q2. I KEEP GETTING 'SYNTAX ERROR IN 340' WHEN USING 'DWNLOD.VKY'ON MY UNEXPANDED VIC20.A2. THIS APPEARS TO BE ONE OF THE MOST COMMON PROBLEMS.IT SEEMS THAT A NUMBER OF PEOPLE USE '?' AS AN ABBREVIATION FOR'PRINT' WHEN TYPING IN PROGRAMS. WHILE '?' IS AN ABBREVIATIONFOR 'PRINT', IT IS NOT (REPEAT NOT) AN ABBREVIATION FOR 'PRINT#'.FREQUENTLY WHEN THIS HAPPENS,PEOPLE THINK THAT THE ERROR IS INTHE PART OF THE LINE WHICH SAYS 'IFMTHEN'. MOST PEOPLE AREUSED TO HAVING ONE OR MORE OPERATORS (IE '=' OR '<' OR '>'  )AND THEN ANOTHER NUMBER (OR VARIABLE) FOLLOWING THE VARIABLE.EXAMPLE- 'IFM=0THEN'THIS IS NOT NECESSARY. IF YOU DO NOT INCLUDE ANYTHING AFTERTHE VARIABLE, THE COMPUTER TREATS IT AS THOUGH '<>0' (DOES NOTEQUAL ZERO) FOLLOWS THE VARIABLE. 'IFMTHEN' IS IDENTICALTO 'IFM<>0THEN' EXCEPT THAT THE FORMER USES LESS MEMORY (VERYIMPORTANT IF USING AN UNEXPANDED VIC) AND IT ALSO RUNS FASTER.Q3. EVERYTHING SEEMS TO WORK OK BUT EVERYTIME I AM SAVING ONTOCASETTE, I LOSE A COUPLE CHARACTERS, OR ELSE THEY BECOME GARBELED.A3. THERE ARE SECTIONS IN EACH DOWNLOADING PROGRAM THAT ARESPECIFICALLY FOR KEEPING YOU FROM LOSING AND GETTING GARBELEDCHARACTERS (ASSUMING THAT THE PROBLEM IS NOT CAUSED BY PHONELINE NOISE). THESE ARE LINES 550 TO 690 IN DWNLOD.VKYLINES 550 TO 610 AND LINE 990 IN UPLOD4.VKYLINES 440 TO 510 IN PARTI6.VKYI WOULD SUGGEST THAT YOU CHECK THESE LINES AGAINST WHAT IS INTHE FILE

FOR ACCURACY.Q4. I AM RATHER NEW TO COMPUTING AND NOT SURE OF MANY THINGS.I AM NOT SURE HOW TO USE YOUR PROGRAM TO DOWNLOAD.A4. PLEASE LOOK AT MY OTHER FILES (VKY???.*) FORINSTRUCTIONS ON HOW TO USE THE PROGRAM. IF YOU ARE NOT SUREWHAT DOWNLOADING OR UPLOAD MEANS,LOOK AT THE FILE 'HELP.HLP'FOR A BRIEF EXPLANATION OF DOWNLOADING AND UPLOADING AND WHERE TO FIND MOREONLINE DOCUMENTATION FOR THIS SIG.Q5. I AM NOT SURE HOW TO USE YOUR PROGRAM DWNLOD.VKY  DO I LOAD IT BEFORE OR AFTER I LOAD (INSERT THE NAME OF YOURFAVORITE PROGRAM HERE).A5. NEITHER-YOUR COMPUTER CAN ONLY RUN ONE PROGRAM AT A TIME,NOT TWO.MY PROGRAMS ARE NOT DESIGNED TO BE ADDED ONTO (OR WITH)ANOTHER PROGRAM THAT YOU HAVE IN YOUR COMPUTER.Q6. IS THERE ANY WAY TO CHANGE YOUR PROGRAM(S) TO HAVE 40 COLUMNSON THE SCREEN? ALSO,HOW CAN I SET/CHANGE THE DEFAULT VALUESFOR THE BAUD,PARITY,ETC.?A6. THE ONLY WAY I KNOW OF TO GET 40 CHARACTERS PER LINE ISTHRU MACHINE LANGUAGE PROGRAMMING. BASICALLY,IF YOU KNOWHOW TO DO MACHINE LANGUAGE PROGRAMMING,YOU PROBABLY WOULDN'TEVEN ASK THE QUESTION, AND IF YOU DON'T KNOW MACHINE LANGUGE,YOU'VE GOT QUITE A WAYS TO GO JUST TO BE ABLE TO UNDERSTANDHOW FORMIDABLE A TASK YOU ARE ASKING ABOUT.AS FAR AS CHANGING THE BAUD RATE, PARITY, ETC. THAT IS COVEREDIN DETAIL IN THE VIC PROGRAMMERS REFERENCE GUIDE UNDER THESECTION LABELED 'RS-232 INTERFACE'. AGAIN, IF YOU ARE APROGRAMMER,YOU PROBABLY WOULDN'T BE ASKING THAT QUESTION(BECAUSE YOU'D PROBABLY ALREADY HAVE THE BOOK,READ IT, ANDWOULD ALREADY KNOW ABOUT IT),AND IF YOU'RE NOT A PROGRAMMER,THEN YOU WOULDPROBABLY MESS THINGS UP BY CHANGINGTHINGS THE WRONG WAY. IT IS BEYOND MY SCOPE AT THIS POINT ANDTIME TO GIVE PROGRAMMING LESSONS. (IE. IT'S NOT A SIMPLEMATTER). ESSENTIALLY WHAT IT BOILS DOWN TO IS IF YOU KNOWENOUGH PROGRAMMING TO BE ABLE TO LOOK UP THE INFORMATION INA BOOK TO FIND OUT HOW IT IS DONE, YOU WOULDN'T ASKIF YOU HAVE TO ASK,THEN YOU PROBABLY WOULDN'T UNDERSTAND THEANSWER.(IT'S A LITTLE LIKE ASKING HOW TO BUILD A HOUSE-IF YOU WANT TO BUILD A HOUSE AND DON'T KNOW HOW, YOU'RE GOINGTO HAVE A HARD TIME FINDING SOMEBODY TO EXPLAIN TO YOUHOW TO DO IT. (IT'S SOMETHING THAT TAKES A WHILE TO LEARN).(IF THIS WERE A MAGAZINE ARTICLE,THIS IS THE POINT AT WHICHTHE AUTHOR WOULD WRITE 'THAT IS BEYOND THE SCOPE OF THIS ARTICLE).Q7. HOW DO I MAKE YOUR PROGRAM WORK? I AM FAIRLY NEW TOCOMPUTERS AND ALTHOUGH I HAVE READ YOUR INSTRUCTIONS, INEED MORE SPECIFIC INSTRUCTIONS.A7. I'LL TRY TO MAKE THE INSTRUCTIONS AS SIMPLE AS POSSIBLE::::AND LEAVE NOTHING OUT-THIS IS AS EXPLICIT AS I CAN BE.....     A. TURN ON YOUR COMPUTER     B. LOAD THE PROGRAM (EXAMPLE-   LOAD"DWNLOD"    )     C. TYPE IN 'RUN'     D. DIAL YOUR COMPUSERVE PHONE NUMBER     E. WHEN THE RED LIGHT ON THE SIDE OF YOUR MODEM TURNS ON        (TO INDICATE THAT YOU ARE CONNECTED UP TO COMPUSERVE)        PUSH THE F1 KEY (YOU MIGHT HAVE TO PUSH IT MORE THEN ONCE)      G. THE SCREEN SHOULD SAY '^C' AND THEN SHOULD SAY 'USER ID:'        (IF YOU CALL THRU TYMNET, ETC. THEY HAVE DIFFERENT LOGON        PROCEDURES-CONSULT COMPUSERVE AS TO HOW TO LOGON THRU OTHER        NETWORKS TO COMPUSERVE)     H. THAT IS BASICALLY IT- YOU ESSENTIALLY USE THE SAME TYPE OF        CONNECTION PROCEDURE YOU USED TO LOGON TO READ THIS.     I. TO DOWNLOAD SOMETHING,YOU USE THE F5 AND THE F7 KEY TO        TURN ON AND OFF (RESPECTIVELY) THE DOWNLOADING FUNCTIONS.        IF YOU DON'T KNOW WHAT DOWNLOADING MEANS, THEN PLEASE LOOK        AT THE FILE NAMED 'HELP.HLP' FOR A SIMPLE EXPLANATION        OF WHERE THE DOWNLOADING AND UPLOADING INSTRUCTIONS ARE.Q8. THE SOFTWARE I AM USING SUPPORTS 40 COLUMNS. HOW CAN IUSE YOUR PROGRAMS (OR PARTS OF YOUR PROGRAMS) WITH THESOFTWARE I AM USING TO DOWNLOAD?A8. I DON'T KNOW. THE PROGRAMS AS WRITTEN WILL RUN AS THEY ARE.I CANNOT TELL YOU HOW TO MODIFY A PROGRAM I HAVE NEVER BOUGHTAND THEREFORE HAVE NEVER USED AND THEREFORE REALLY KNOWNOTHING ABOUT.WQ9. WHEN I USE YOUR PROGRAMS TO TRY TO SEND PROGRAMS TO AFRIEND OVER THE TELEPHONE LINES,THEY ONLY GET THE FIRST(BEGINNING) PART OF WHAT I SEND-EVERYTHING ELSE IS LOST.A9. SOMEONE ASKED ME ABOUT THAT LAST YEAR AND I GAVE THEMA MODIFICATION TO THE PROGRAM FOR THAT. (I DON'T REMEMBEROFFHAND IF THEY GOT BACK TO ME TO LET ME KNOW IF IT WORKED).I'LL TRY TO LOOK UP (OR REWRITE) THOSE MODIFICATIONS AGAINAND THEN PUT THEM IN THE DATABASE.Q10. THE PROGRAM WORKS FINE. THE ONLY PROBLEM I HAVE IS THATI DON'T KNOW HOW TO MAKE IT DOWNLOAD.A10. YOU USE THE F5 AND THE F7 FUNCTION KEYS.----NOTE---- I RECIEVE MANY QUESTIONS SIMILAR TO THIS ONE.THE ANSWER IS FOUND IN THE INSTRUCTIONS FOR THE PROGRAM.IT WOULD APPEAR AS THOUGH SOME PEOPLE COPY THE DOWNLOADPROGRAM TO USE, BUT NEGLECT TO LOOK AT THE INSTRUCTION FILETHAT EXPLAINS HOW TO USE IT. REMEMBER, IF ALL ELSE FAILS,READ THE INSTRUCTIONS. DO NOT ASSUME THAT ALL THE OTHER FILESARE PROGRAMS-THEY ARE NOT!!!!!!!!!SOME ARE INSTRUCTIONS, SOME ARE TEXT FILES EXPLAINING THINGS.WHAT I FIND "INTERESTING" IS THAT WHEN I CHECK MY FILES, ITSEEMS THAT ABOUT TWICE AS MANY PEOPLE HAVE ACCESSED THEPROGRAM FILE AS THE INSTRUCTIONS. THAT IMPLYS THAT ABOUTHALF THE PEOPLE WHO READ THE PROGRAM FILE DID NOT LOOK ATTHE INSTRUCTIONS FOR USING IT.Q11. IS THERE ANY WAY I CAN MODIFY YOUR PROGRAMS SO I CANDOWNLOAD TO DISK INSTEAD OF CASETTE?A11. YES, BUT I'VE FOUND IT EASIER TO JUST WRITE ANOTHERPROGRAM FOR THE DISK (WHEN YOU HAVE A DISK, YOU RARELY USETHE CASETTE). I'VE MISPLACED MY COPY OF THAT MESSAGE (FROMLAST YEAR) AND SO I CAN'T SEND THAT PERSON A MESSAGE BACKTHAT I NOW HAVE A PROGRAM TO DO THIS.*********MISC. *******I CAN ONLY ANSWER QUESTIONS REGARDING EQUIPMENT THAT I AMFAMILAR WITH (AT THE TIME OF THAT LAST QUESTION, I DID NOTHAVE A DISK DRIVE.) THE EQUIPMENT THAT I CURRENTLY (APRIL 1984)HAVE IS-- VIC20,16K EXPANSION,1525 PRINTER,1541 DISK DRIVE,DATASETTE RECORDER,1600 VICMODEM.**********IF THE PROGRAM YOU ARE TRYING TO USE HAS AN ERROR, PLEASEGO BACK AND LOOK AT THE FILE AGAIN. IT'S VERY POSSIBLE THATWHEN YOU LOOKED AT IT THE FIRST TIME, YOU COULD HAVE GOTTENSOME PHONE LINE NOISE WHICH GARBELED THE PROGRAM LINE(IE. YOU NOW HAVE AN INCORRECT LINE WITH ERROR(S) IN IT ).COMPARE WHAT YOU HAVE TO WHAT IS ACTUALLY THERE IN THE FILE.THIS WILL CAUSE THE PROBLEM TO SHOW UP (IF IT WAS A PHONELINE NOISE PROBLEM.*******************FOR THOSE WHO ARE INTERESTED-----I STARTED USING THE PROGRAM THAT CAME IN THE INSTRUCTIONBOOK WITH THE MODEM. AFTER FINDING OUT ABOUT DOWNLOADING,I DECIDED TO CHANGE IT FOR DOWNLOADING-IT TOOK ABOUT SIX MONTHSOF TRIAL AND ERROR (LITTLE THINGS I DIDN'T KNOW ABOUT) TOGET IT TO WORK.I WOULD LIKE TO THANK THOSE WHO DIRECTLY AND INDIRECTLYHELPED IT GET WRITTEN IN THE FIRST PLACE (THAT I KNOW ABOUT).JOE BROWNNEIL HARRISJEFF HANDJIM BUTTERFIELDIF I LEFT OUT ANYBODY PLEASE FORGIVE MY LACK OF KNOWLEDGE.THANK YOUTERRY IMLER [73465,1635]END OF TEXT