SIDNP2.DOC Sidplayer News #4 part two...MISCELLANEOUS1. Be very careful to never let two voices using the same waveform play notesof the exact same pitch at the very same time.  When this is allowed tohappen, a phenomenon known as "destructive harmonic interference" can occur,which in no way damages your computer but can cause the two voices to canceleach other out and be inaudible.  The effect happens randomly and is notalways noticeable.  The only time that two voices should simultaneously playthe exact same pitches in the same octave with the same waveform is when thevoices are being detuned.2. If two notes of the same pitch are tied together, it is a good practice toput the shorter note first.  This is because a note tied to another note doesnot release.  If the longer note is put first, the release cannot begin untilthe shorter note starts playing, and in some cases the release would normallybegin sooner.  This is especially true when larger release points are used.Putting the shorter note before the longer note ensures that the release willbegin at the appropriate place.3. When a grace note is played, remember that the duration for the grace noteshould come out of the duration for the preceding note.  This way, the notefollowing the grace note will begin right on the beat, as it should, insteadof the grace note.  It may be necessary to use utility durations for both thegrace note and the note preceding it.4. KPLAY and MPLAY and the other players end a song right when the lastduration is complete, even if some of the voices are not done releasing.  Tolet each voice fade out and finish completely, it is a nice touch to put arest after the last note in each voice.5. Remember that when you are creating a very long song, it is much easier tobreak the song into parts, enter and edit each part separately, and thencombine them using the MERGE utility found on page 369.  This utility can alsobe used to create songs larger than what the Editor can hold in memory.6. The LISTER utility on page 365, or the revised version called PRINTER whichprints in three columns only to a printer, can be used to find duration bugsin songs.  See SIDNEWS#1 for information on the PRINTER utility.7. Whenever possible, please try not to use the default waveform and envelope.The default settings were chosen because they give a simple, neutral waveformand envelope which can be modified easily.  Playing all three voices with thedefault square wave and organ-type envelope, however, can make any song, evena very good one, sound pretty blah.8. Don't overlook the subtle but significant effects that different envelopesettings can have on a song.  Envelopes seem to be one of the most ignoredaspects of songs.MASTER COMPOSER SYNDROMEIt has been observed that people who formerly used the Master Composer musicsystem and now use Sidplayer exhibit a strong tendency to use a heavy dose ofvibrato and pulse width sweeping in their songs.  Presumably this is becausethese are two easy-to-use features that are not found on Master Composer.  Ifyou might be such a person, we urge you to control your fascination with thesefeatures and use them sparingly.  Thank you!THE BASEBALL CARD PHENOMENONSome music systems may make songs sound better than other music systems, butthe quality of a song depends just as much, if not more so, on the time andcare taken by the person entering the song.  While it is nice to have lots ofsongs to collect and trade, it is also important to keep the quality levelhigh in order to maintain interest in the songs.  We would prefer to encouragepeople to emphasize quality over quantity.SIDPLAYER ON YOUR STEREOIf you don't have your computer hooked up to a stereo, you're only gettingpart of the story when you play a song.  Playing songs over a stereo gives youfuller bass lines and triangle waveforms, and reveals other things you neverknew were there.  See SIDNEWS#1 for instructions on how to hook your computerup to a stereo.SOME ADVANCED TECHNIQUES TO PONDER1. The sync mode is not used very often because it requires two voices,leaving only one voice free.  The mode offers a whole variety of new tonetypes, however, and is definitely worth investigating.  Try the sync mode withdifferent combinations of waveforms and TPS values, as demonstrated by thesong TPI#14.2. See page 337 for a discussion of using sync and ring modulation at the sametime.  With this technique, you can sometimes get tones that sound like theyare being filtered, even though they are not.3. Another possibility is to use portamento with the sync mode.  This is doneby putting an absolute set command of a different pitch before each note thatis played.  When this is done on the synchronized voice which adds theharmonics, an interesting "wah" effect is produced, as in the song ETAL.4. Some people have gotten interesting results by using the sync or ringmodulation modes with nonstandard configurations.5. Absolute set commands can be placed between rests following a note with alarge RLS value to make a note slowly fade away while changing pitch.  Thistechnique was used in the song STUNTMAN.6. A subtle technique is to use the absolute set command with detuning andportamento.  Turn on the detuning and set a detuned pitch with the absoluteset command.  Then turn off the detuning and play a note with a slowportamento rate in effect.  The note will not start at exactly the correctpitch but will move to the desired pitch.  This may be useful for simulatingsome vocal effects.  A variation on this method is to not use detuning andjust use the absolute set command to set the pitch one half step below thedesired pitch, with a faster portamento rate.7. Detuning and transposition can be an interesting combination.  Octavetranspositions with detuning were used in PIPERS, CALLIOPE, BISTRO, andAMERICA.8. An advanced use of detuning is to slightly detune a voice, even when it isnot being played along with another voice containing the same notes, for afuller sound.  This technique is based on a principle used by barbershopquartets.9. The POLYCON utility can be used to add a pedal effect or lingering effect(like a bell) to a song, as in FUER.  This technique can also be used withfast arpeggios for another interesting effect.  See SIDNEWS#3 for fullinstructions.10. Detuning with polyphony also gives an interesting effect.  Thiscombination was used in the song ISLAND.11. When the waveform TP is used, pulse width sweeping can be used to sweepthe width past 2048, thereby making the volume fade out.12. A note by note use of utility duration is very tedious but can producesome wonderful phrasing effects.  See the song FUER for an example.  Evenusing just a few carefully placed utility durations can add a nice, morenatural touch to a song.13. When two voices are playing similar notes, perhaps with different pulsewidths or detuning, a very short utility duration rest can be placed at thebeginning of one of the voices to create an echo effect.  See the version ofTAKE FIVE by Stan Halaby for an example.14. While having only three voices can be quite a limitation, it is sometimespossible to use one voice for two different effects simultaneously.  Forexample, CANTINA uses the same voice for both bass notes and percussioneffects.  The bass notes are played on the beat and the percussion effects areplayed on the offbeat.  Phrase calls can make it easy to alternate betweendifferent waveform and envelope settings.15. One technique to get around volume problems on a voice is to release anote before has completed its attack phase.  This will require someexperimentation with attack rates and release points.  The technique was usedin the stereo version of AXEL F.16. A rapid succession of different pitches using white noise makes aninteresting percussion line, as in the song IBEX.17. Longer white noise notes, as in the song OSPREY, can also give aninteresting effect.18. One other interesting effect is to play a sequence of notes of the samepitch but with different pulse widths.FINAL REMARKS1. Some of these suggestions and techniques are described in the book, alongwith complete instructions on how to use the Sidplayer Editor and a fairlycomprehensive introduction to music theory.2. Most people feel that it is worth $16.95 to get the complete documentation.3. Other music systems cost twice as much and are not as well documented.4. The book also has bitmapped graphics and sprite utilities, and sections onAdvanced BASIC.5. The Sidplayer Editor is not a Public Domain program and is supposed to beused only by those who have purchased Volume Two.If you use the Sidplayer Editor and don't have the book...