C64S - Commodore 64 Software Emulator

Shareware verison 0.9B, November 1993

(C) 1991,92,93 Miha Peternel

USER'S MANUAL

___________________

1. INTRODUCTION

Welcome to a new emulator of the good old fat C64. It will allow

you to run most of the original C64 software on your PC.

This version is SHAREWARE. It is free for personal use, but if

you want a perfect version of the emulator, you will need to

register.

Minimum configuration:

- 386 or higher (286 in the demo version only)

- 640K RAM

- VGA display (register compatible)

- DOS 3.3 (lower not tested)

Optional:

- a soundcard:

Gravis UltraSound

Sound Blaster (all versions)

Covox or compatible DAC (digital to analog converter)

- analog PC joystick

- tape interface (supported in registered version)

- disk 1541 interface (supported in registered version)

You need at least a 386DX40 with a fast VGA for real time

emulation. A soundcard is recommended, though the sound can also

be played through PC speaker (at rather low quality).

________________

2. IN ACTION

The emulator emulates all the components of C64 needed to make

the programs run correctly, display genuine graphics, synthesize

music, and emulate I/O operations.

The keyboard layout is original C64 with some exceptions due to

different PC layout.

Some special emulator features make the work with the emulator

easier. The tapes and disks to be used with the emulator are

stored in a special format in image files on the hard disk. You

can select the tape position or a disk in the selection screen.

Emulation settings can be changed in the option screen.

The following keys have special meaning:

- F8 enters option screen

- F9 enters selection screen

- F10 turns emulation of both joystick ports on

- ALT+F10 recalibrates analog joysticks

- ESC exits emulator screens

- CTRL+BREAK exits the emulator

- CTRL+ALT+DEL or CTRL+ALT+BackSpace emulate reset

- CTRL+ALT+INS causes brutal reset (for reset-protected programs)

Table of replaced keys with their equivalents:

- F1/F2..F7/F8      F1..F4  (will be changed to F1..F8)

- C=                Alt

- Ins/Del           BackSpace

- Clr/Home          Home

-                 Insert

-                  Delete

- Run/Stop          Tab

- Restore           F12

Keys used for joystick emulation:

- Port A (2):

W    up

S    down

U    left

I    right

O    fire

- Port B (1): (numeric keys)

7    up

4    down

8    left

9    right

+    fire

________________________

3. FOR THE BEGINNERS

Emulator exists of two executables:

- C64S     (for 386 and higher)

- C64S286  (for 286 and higher)

Run C64S286 only if you have a 286.

The following options can be used on the command line when you

run the emulator:

1. SOUND options:

-s0  for no sound output (default)

-su  for Gravis UltraSound (220)

-sb  for Sound Blaster (220)

-sd  for DAC at LPT1 (278)

-sp  for PC speaker (042)

If your soundcard is set to a nonstandard base address, you need

to specify it with the sound option on the command line.

Example: If your Gravis UltraSound card is set to base port

address 240, use -su240 on the command line. LPT base addresses

for DAC are 278 (LPT1), 378 (LPT2) and 3BC (LPT3). You may use a

different address if you are not using LPT DAC.

2. JOYSTICK options:

-j0  for no analog joystick (default)

-j1  for one analog joystick

-j2  for two analog joysticks

If you want to use analog joystick, you need to recalibrate it by

pressing ALT-F10 in the emulator. Joystick must be centered when

pressing ALT-F10 in order to recalibrate it correctly. The

information will be saved to configuration file for later use.

The options specified on the command line will be written to the

configuration file so you don't need to rewrite them the next

time you run the emulator. You only need to specify them when you

reconfigure your computer or when you want to force different

functionality of the emulator.

Fast intro:

1. Run the emulator with appropriate options (described above).

2. If you use analog joystick, center it and press ALT+F10 to

recalibrate it.

3. Press F9 to get to the selection screen, select a tape with

cursor keys and confirm with Enter, select a file and again

confirm with Enter. The tape pointer will now point to selected

tape and file and you will be returned to the C64 screen.

4. Press SHIFT+TAB (SHIFT+RUN/STOP), which will load a tape

program and run it automatically.

5. Press F10 to turn joystick emulation on.

6. Enjoy the nostalgia. :)

7. You can always press F8 to enter option screen, where you can

adjust some parameters to make the emulation at you computer's

speed more acceptable. It is especially important to play with

the raster and refresh speeds if you want to reproduce rhythmic

music.

Pressing F9 in the selection screen will switch between tape and

disk indexes. Selecting a disk and pressing Enter works just the

same as if you put a new diskette into the floppy drive. By

default the emulator will read TESTDISK.D64. Be careful if the

program writes to disk, because it can damage the contents of the

disk you switch to when writing.

Using 1541 drive operates in the original way. The CBM DOS

commands (except format) work, too. Write LOAD"$",8,1 to load

directory, LIST to see it, LOAD"prg name",8 to load a program

(you may use wildcards) and RUN to start it. To load a file to

its original address, you need to use LOAD"prg name",8,1. Run a

machine code program writing SYS xxxx, where xxxx is the program

entry address (decimal). VIC 20 emulator is loaded by writing

LOAD"VIC*",8,1 and started with SYS49152.

The emulator will always scan its directory for .T64 and .D64

files. If you add new files to the emulator's directory, they

will automatically appear in the corresponding selection screen.

* The emulator does not support changing directories yet. All its

files need to be in the same directory.

Emulating C64 in all aspects also means you can find yourself in

the same trouble as you were using a real C64 if something went

wrong, moving joystick or pressing fire will dump characters to

screen, per example.

You also need to think about the emulator's features. Turning

keyboard joystick emulation on means that used keys won't work

like they should if you try to write text. You need to turn

joystick emulation off (in the option screen).

If you have difficulties using analog joystick, press ALT-F10 to

recalibrate it.

Sometimes when a single analog joystick emulates both joystick

ports it may appear that the game doesn't recognize some joystick

directions (like in Green Beret). The problem is easily solved by

assigning analog joystick port emulation to one port only.

____________________

4. COMPATIBILITY

The following is the compatibility list.

+ means implemented and compatible

- means not implemented or not compatible

* means planned in the future

CPU

+ 6510 documented instruction set (JMP (indirect) fixed)

* 6510 undocumented instructions only partially implemented

+ $01 memory banking

+ flags (not 100% compatible in decimal operations)

* cycle counts

VIC

+ all graphic modes

+ black screen

+ sprites

+ sprite to background priority

+ sprite to background collision

- sprite to sprite collision buggy

+ raster interrupts

+ multiplied sprites

+ hardware scroll

+ bad line scroll

+ shrunk screen

+ 8 line update

* line by line update

* forced DMA

* character line redraw

* border and border tricks in X-mode

* cycle exact screen changes

SID

+ 3 channels

+ 8 bit sampled original waveforms

+ ADSR envelope control

+ master volume control

+ pulse width modulation

- no channel to channel effects

- no filters

* filtered waveforms may be implemented on GUS

CIA 1

+ keyboard emulation (still needs improvements)

+ joystick emulation

+ timer A (real time)

- timer B

* time of day (partially implemented)

- serial port

+ IRQ control

* rebindable keyboard

CIA 2

+ VIC bank select

- other features not implemented

1541

+ ROM level emulation (16K RAM & I/O + 16K ROM)

+ supports CBM DOS commands except format

+ fast (ROM trapped) C64 to 1541 connection

+ image file operation

- protection support (GCR coding)

* real 1541 connection

* improved (GCR coded) image file format

* fast loader support

* PC side full disk transfer utilities

TAPE

+ ROM trapped tape routines

+ tape image files with directory

+ ultra fast loading

+ Turbo Tape loader for PC tape connector

+ MakeTape utilities

* built-in tape tools for comfortable tape image manipulation

* SB and GUS versions of Turbo Tape loader

_____________________

5. FILE TRANSFERS

Tape files in the Turbo Tape format are the easiest to transfer

using my PC loader. The connector scheme is simple and will be

published together with the loader in the registered version.

IMPORTANT !

Since all beta tester haven't got my MakeTape utility, a utility

named CONV64 has appeared, which creates FALSE .T64 files. The

start and end address are not set correctly, that's why many

compressed programs refused to work. Please use MakeTape for best

results. There's also a C source included in case someone would

like to tailor it to his/her needs.

Disk files are possible to be transferred using X1541 program and

a special 1541 to LPT cable. All the required schemes are a part

of X1541. The files transferred in this way can be converted to

tape files with MakeTape program or (if the whole disk contents

is transferred) used as a disk image.

I'll also support this type of 1541 connection in the registered

version of my emulator.

________________

6. UTILITIES

The following utilities are planned to go with the emulator:

- built-in debugger

- Turbo Tape loader with support for SB and GUS inputs

- MakeTape formatter / importer / exporter

- 1541 disk transfer utility

- MakeDisk formatter / importer / exporter

- 6510 Cross Assembler

______________

7. RELEASE

The 1.0 release won't be ready in November, because I have

decided (with a lot of help from beta testers) to add a long list

of features not planned previously, not to mention the university

after all. Please understand that the emulator is written in 100%

assembler, which makes me write the code very carefully in order

to avoid bugs and allow further enhancements. I also wish I had

more time to answer all the mail and email I receive.

IMPORTANT !

Please DO NOT register the emulator until I announce V 1.0 is

ready. I may arrange the registered version to be distributed by

a software distributing company in which case it could also be

shipped with special cables needed for 1541 and tape connection.

_______________

8. CONTACTS

E-mail:

miha@rsc3.hermes.si (slow reception, fast reply)

miha.peternel@abm.gn.apc.org (fast reception, slow reply)

E-mail messages to both addresses if possible.

Phone:

+386-64-311-959 (fax/modem on demand)

(How come this number reminds me of something ?)

Snail mail:

Miha Peternel (for C64S)

Pot v Bitnje 66

64000 Kranj

Slovenia, Europe